  - Relevant identified uses of the substance or mixture and uses advised against:

Relevant uses: Car repair; hardener for coatings. For professional users only. Uses advised against: All uses not specified in this section or in section 7.3

  - Details of the supplier of the safety data sheet:

Rags Ltd.

Durbes str. 8, Riga, LV 1007

Latvija

www.rags.lv

+37167808780

rags@rags.lv

1.4	Emergency telephone number: +371 67042473.; 112

STONDER Starline VHS Crystal Clearcoat Hardener Standart

NGE7-M7RW-400W-C9Q4

1.1	Product identifier:

Other means of identification: UFI:

SECTION 1: IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

# SECTION 2: HAZARDS IDENTIFICATION

  - Classification of the substance or mixture: CLP Regulation (EC) No 1272/2008:

Classification of this product has been carried out in accordance with CLP Regulation (EC) No 1272/2008.

Acute Tox. 4: Acute inhalation toxicity, Category 4, H332

Aquatic Chronic 3: Hazardous to the aquatic environment, long-term hazard, Category 3, H412 Eye Irrit. 2: Eye irritation, Category 2, H319

Flam. Liq. 3: Flammable liquids, Category 3, H226 Skin Irrit. 2: Skin irritation, Category 2, H315

Skin Sens. 1: Sensitisation, skin, Category 1, H317

STOT RE 2: Specific target organ toxicity — Repeated exposure, Hazard Category 2 (Oral), H373 STOT SE 3: Respiratory tract toxicity, single exposure, Category 3, H335

STOT SE 3: Specific toxicity causing drowsiness and dizziness, single exposure, Category 3, H336

  - Label elements:

CLP Regulation (EC) No 1272/2008:

Warning

Hazard statements:

Acute Tox. 4: H332 - Harmful if inhaled.

Aquatic Chronic 3: H412 - Harmful to aquatic life with long lasting effects. Eye Irrit. 2: H319 - Causes serious eye irritation.

Flam. Liq. 3: H226 - Flammable liquid and vapour. Skin Irrit. 2: H315 - Causes skin irritation.

Skin Sens. 1: H317 - May cause an allergic skin reaction.

STOT RE 2: H373 - May cause damage to organs through prolonged or repeated exposure (Oral). STOT SE 3: H335 - May cause respiratory irritation.

STOT SE 3: H336 - May cause drowsiness or dizziness.

Precautionary statements:

  - Substance:

Non-applicable

  - Mixture:

Chemical description: Mixture composed of chemical products

Components:

In accordance with Annex II of Regulation (EC) No 1907/2006 (point 3), the product contains:

⁽¹⁾ Substances presenting a health or environmental hazard which meet criteria laid down in Regulation (EU) No. 2020/878

To obtain more information on the hazards of the substances consult sections 11, 12 and 16.

SECTION 3: COMPOSITION/INFORMATION ON INGREDIENTS

4.1	Description of first aid measures:

SECTION 4: FIRST AID MEASURES

P210: Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. P261: Avoid breathing dust/fume/gas/mist/vapours/spray.

P271: Use only outdoors or in a well-ventilated area.

P280: Wear protective gloves/protective clothing/respiratory protection/eye protection/protective footwear. P302+P352: IF ON SKIN: Wash with plenty of water.

P304+P340: IF INHALED: Remove person to fresh air and keep comfortable for breathing. P312: Call a POISON CENTER/doctor if you feel unwell.

P403+P233: Store in a well-ventilated place. Keep container tightly closed.

P501: Dispose of contents/container in accordance with regulations on hazardous waste or packaging and packaging waste respectively.

Supplementary information:

EUH204: Contains isocyanates. May produce an allergic reaction.

Substances that contribute to the classification

Hexamethylene diisocyanate, oligomers; N-butyl acetate; Xylene; Hydrocarbons, C9, aromatics

Additional Labelling:

As from 24 August 2023 adequate training is required before industrial or professional use.

2.3	Other hazards:

Product fails to meet PBT/vPvB criteria

Endocrine-disrupting properties: The product fails to meet the criteria.

SECTION 2: HAZARDS IDENTIFICATION (continued)

| Identification |  | Chemical name/Classification |  |  | Concentration |
| --- | --- | --- | --- | --- | --- |
| CAS: | 28182-81-2 | Hexamethylene diisocyanate, oligomers⁽¹⁾ |  | Self-classified |  |
| EC: Index: REACH: | 931-274-8 Non-applicable 01-2119485796-17- |  |  |  | 50 - <75 % |
|  |  | Regulation 1272/2008 | Acute Tox. 4: H332; Skin Sens. 1: H317; STOT SE 3: H335 - Warning |  |  |
|  | XXXX |  |  |  |  |
| CAS: | 123-86-4 | N-butyl acetate⁽¹⁾ |  | ATP CLP00 |  |
| EC: Index: REACH: | 204-658-1 607-025-00-1 01-2119485493-29- |  |  |  | 10 - <25 % |
|  |  | Regulation 1272/2008 | Flam. Liq. 3: H226; STOT SE 3: H336; EUH066 - Warning |  |  |
|  | XXXX |  |  |  |  |
| CAS: | 110-43-0 | heptan-2-one⁽¹⁾ |  | ATP CLP00 |  |
| EC: Index: REACH: | 203-767-1 606-024-00-3 01-2119902391-49- |  |  |  | 10 - <25 % |
|  |  | Regulation 1272/2008 | Acute Tox. 4: H302+H332; Flam. Liq. 3: H226 - Warning |  |  |
|  | XXXX |  |  |  |  |
| CAS: | 1330-20-7 | Xylene⁽¹⁾ |  | Self-classified |  |
| EC: Index: REACH: | 215-535-7 601-022-00-9 01-2119488216-32- XXXX |  |  |  | 10 - <25 % |
|  |  | Regulation 1272/2008 | Acute Tox. 4: H312+H332; Aquatic Chronic 3: H412; Asp. Tox. 1: H304; Eye Irrit. 2: H319; Flam. Liq. 3: H226; Skin Irrit. 2: H315; STOT RE 2: H373; STOT SE 3: H335 - Danger |  |  |
| CAS: | 128601-23-0 | Hydrocarbons, C9, aromatics⁽¹⁾ |  | Self-classified |  |
| EC: Index: REACH: | 918-668-5 Non-applicable 01-2119455851-35- |  |  |  |  |
|  |  | Regulation 1272/2008 | Aquatic Chronic 2: H411; Asp. Tox. 1: H304; Flam. Liq. 3: H226; STOT SE 3: H335; STOT SE 3: H336; EUH066 - Danger |  | 10 - <25 % |
|  | XXXX |  |  |  |  |

6.1	Personal precautions, protective equipment and emergency procedures: For non-emergency personnel:

Isolate leaks provided that there is no additional risk for the people performing this task. Evacuate the area and keep out those without protection. Personal protection equipment must be used against potential contact with the spilt product (See section 8). Above all prevent the formation of any vapour-air flammable mixtures, through either ventilation or the use of an inert medium. Remove any source of ignition. Eliminate electrostatic charges by interconnecting all the conductive surfaces on which static electricity could form, and also ensuring that all surfaces are connected to the ground.

For emergency responders:

SECTION 6: ACCIDENTAL RELEASE MEASURES

  - Extinguishing media: Suitable extinguishing media:

If possible use polyvalent powder fire extinguishers (ABC powder), alternatively use foam or carbon dioxide extinguishers (CO₂).

Unsuitable extinguishing media:

IT IS RECOMMENDED NOT to use full jet water as an extinguishing agent.

  - Special hazards arising from the substance or mixture:

As a result of combustion or thermal decomposition reactive sub-products are created that can become highly toxic and, consequently, can present a serious health risk.

  - Advice for firefighters:

Depending on the magnitude of the fire it may be necessary to use full protective clothing and self-contained breathing apparatus (SCBA). Minimum emergency facilities and equipment should be available (fire blankets, portable first aid kit,...) in accordance with Directive 89/654/EC.

Additional provisions:

Act in accordance with the Internal Emergency Plan and the Information Sheets on actions to take after an accident or other emergencies. Eliminate all sources of ignition. In case of fire, cool the storage containers and tanks for products susceptible to combustion, explosion or BLEVE as a result of high temperatures. Avoid spillage of the products used to extinguish the fire into an aqueous medium.

SECTION 5: FIREFIGHTING MEASURES

The symptoms resulting from intoxication can appear after exposure, therefore, in case of doubt, seek medical attention for direct exposure to the chemical product or persistent discomfort, showing the SDS of this product.

By inhalation:

Remove the person affected from the area of exposure, provide with fresh air and keep at rest. In serious cases such as cardiorespiratory failure, artificial resuscitation techniques will be necessary (mouth to mouth resuscitation, cardiac massage, oxygen supply, etc.) requiring immediate medical assistance.

By skin contact:

Remove contaminated clothing and footwear, rinse skin or shower the person affected if appropriate with plenty of cold water and neutral soap. In serious cases see a doctor. If the product causes burns or freezing, clothing should not be removed as this could worsen the injury caused if it is stuck to the skin. If blisters form on the skin, these should never be burst as this will increase the risk of infection.

By eye contact:

Rinse eyes thoroughly with water for at least 15 minutes. If the injured person uses contact lenses, these should be removed unless they are stuck to the eyes, in which case removal could cause further damage. In all cases, after cleaning, a doctor should be consulted as quickly as possible with the SDS for the product.

By ingestion/aspiration:

Do not induce vomiting, but if it does happen keep the head down to avoid aspiration. Keep the person affected at rest. Rinse out the mouth and throat, as they may have been affected during ingestion.

  - Most important symptoms and effects, both acute and delayed:

Acute and delayed effects are indicated in sections 2 and 11.

  - Indication of any immediate medical attention and special treatment needed:

Non-applicable

SECTION 4: FIRST AID MEASURES (continued)

Wear protective equipment. Keep unprotected persons away. See section 8.

  - Environmental precautions:

Avoid at all cost any type of spillage into an aqueous medium. Contain the product absorbed appropriately in hermetically sealed containers. Notify the relevant authority in case of exposure to the general public or the environment.

  - Methods and material for containment and cleaning up:

It is recommended:

Absorb the spillage using sand or inert absorbent and move it to a safe place. Do not absorb in sawdust or other combustible absorbents. For any concern related to disposal consult section 13.

  - Reference to other sections:

See sections 8 and 13.

SECTION 6: ACCIDENTAL RELEASE MEASURES (continued)

SECTION 7: HANDLING AND STORAGE

  - Precautions for safe handling:

A.- General precautions for safe use

Comply with the current legislation concerning the prevention of industrial risks. Keep containers hermetically sealed. Control spills and residues, destroying them with safe methods (section 6). Avoid leakages from the container. Maintain order and cleanliness where dangerous products are used.

B.- Technical recommendations for the prevention of fires and explosions

Transfer in well ventilated areas, preferably through localized extraction. Fully control sources of ignition (mobile phones, sparks,…) and ventilate during cleaning operations. Avoid the existence of dangerous atmospheres inside containers, applying inertization systems where possible. Transfer at a slow speed to avoid the creation of electrostatic charges. Against the possibility of electrostatic charges: ensure a perfect equipotential connection, always use groundings, do not wear work clothes made of acrylic fibres, preferably wearing cotton clothing and conductive footwear. Comply with the essential security requirements for equipment and systems defined in Directive 2014/34/EC (ATEX 100) and with the minimum requirements for protecting the security and health of workers under the selection criteria of Directive 1999/92/EC (ATEX 137). Consult section 10 for conditions and materials that should be avoided.

C.- Technical recommendations on general occupational hygiene

Do not eat or drink during the process, washing hands afterwards with suitable cleaning products.

D.- Technical recommendations to prevent environmental risks

Due to the danger of this product for the environment it is recommended to use it within an area containing contamination control barriers in case of spillage, as well as having absorbent material in close proximity.

  - Conditions for safe storage, including any incompatibilities:

A.- Technical measures for storage Minimum Temp.:	15 ºC

Maximum Temp.:	25 ºC

Maximum time:	12 Months B.- General conditions for storage

Avoid sources of heat, radiation, static electricity and contact with food. For additional information see subsection 10.5

  - Specific end use(s):

8.1	Control parameters:

Substances whose occupational exposure limits have to be monitored in the workplace (European OEL, not country-specific legislation):

SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION

Except for the instructions already specified it is not necessary to provide any special recommendation regarding the uses of this product.

## SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION (continued)

Directive (EU) 2000/39, Directive 2004/37/EC,Directive (EU) 2006/15, Directive (EU) 2009/161, Directive (EU) 2017/164, Directive (EU) 2019/1831:

| Identification | Occupational exposure limits |  |  |
| --- | --- | --- | --- |
| N-butyl acetate CAS: 123-86-4 EC: 204-658-1 | IOELV (8h) | 50 ppm | 241 mg/m³ |
|  | IOELV (STEL) | 150 ppm | 723 mg/m³ |
| heptan-2-one CAS: 110-43-0 EC: 203-767-1 | IOELV (8h) | 50 ppm | 238 mg/m³ |
|  | IOELV (STEL) | 100 ppm | 475 mg/m³ |
| Xylene CAS: 1330-20-7 EC: 215-535-7 | IOELV (8h) | 50 ppm | 221 mg/m³ |
|  | IOELV (STEL) | 100 ppm | 442 mg/m³ |

DNEL (Workers):

|  |  | Short exposure |  | Long exposure |  |
| --- | --- | --- | --- | --- | --- |
| Identification |  | Systemic | Local | Systemic | Local |
| Hexamethylene diisocyanate, oligomers CAS: 28182-81-2 EC: 931-274-8 | Oral | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Inhalation | Non-applicable | 1 mg/m³ | Non-applicable | 0,5 mg/m³ |
| N-butyl acetate CAS: 123-86-4 EC: 204-658-1 | Oral | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Dermal | 11 mg/kg | Non-applicable | 11 mg/kg | Non-applicable |
|  | Inhalation | 600 mg/m³ | 600 mg/m³ | 300 mg/m³ | 300 mg/m³ |
| heptan-2-one CAS: 110-43-0 EC: 203-767-1 | Oral | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 54,27 mg/kg | Non-applicable |
|  | Inhalation | 1516 mg/m³ | Non-applicable | 394,25 mg/m³ | Non-applicable |
| Xylene CAS: 1330-20-7 EC: 215-535-7 | Oral | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 212 mg/kg | Non-applicable |
|  | Inhalation | 442 mg/m³ | 442 mg/m³ | 221 mg/m³ | 221 mg/m³ |
| Hydrocarbons, C9, aromatics CAS: 128601-23-0 EC: 918-668-5 | Oral | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 25 mg/kg | Non-applicable |
|  | Inhalation | Non-applicable | Non-applicable | 150 mg/m³ | Non-applicable |

DNEL (General population):

|  |  | Short exposure |  | Long exposure |  |
| --- | --- | --- | --- | --- | --- |
| Identification |  | Systemic | Local | Systemic | Local |
| N-butyl acetate CAS: 123-86-4 EC: 204-658-1 | Oral | 2 mg/kg | Non-applicable | 2 mg/kg | Non-applicable |
|  | Dermal | 6 mg/kg | Non-applicable | 6 mg/kg | Non-applicable |
|  | Inhalation | 300 mg/m³ | 300 mg/m³ | 35,7 mg/m³ | 35,7 mg/m³ |
| heptan-2-one CAS: 110-43-0 EC: 203-767-1 | Oral | Non-applicable | Non-applicable | 23,32 mg/kg | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 23,32 mg/kg | Non-applicable |
|  | Inhalation | Non-applicable | Non-applicable | 84,31 mg/m³ | Non-applicable |
| Xylene CAS: 1330-20-7 EC: 215-535-7 | Oral | Non-applicable | Non-applicable | 12,5 mg/kg | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 125 mg/kg | Non-applicable |
|  | Inhalation | 260 mg/m³ | 260 mg/m³ | 65,3 mg/m³ | 65,3 mg/m³ |
| Hydrocarbons, C9, aromatics CAS: 128601-23-0 EC: 918-668-5 | Oral | Non-applicable | Non-applicable | 11 mg/kg | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 11 mg/kg | Non-applicable |
|  | Inhalation | Non-applicable | Non-applicable | 32 mg/m³ | Non-applicable |

PNEC:

| Identification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Hexamethylene diisocyanate, oligomers CAS: 28182-81-2 EC: 931-274-8 | STP | 88 mg/L | Fresh water | 0,127 mg/L |
|  | Soil | 53183 mg/kg | Marine water | 0,013 mg/L |
|  | Intermittent | 1,27 mg/L | Sediment (Fresh water) | 266701 mg/kg |
|  | Oral | Non-applicable | Sediment (Marine water) | 26670 mg/kg |
| N-butyl acetate CAS: 123-86-4 EC: 204-658-1 | STP | 35,6 mg/L | Fresh water | 0,18 mg/L |
|  | Soil | 0,09 mg/kg | Marine water | 0,018 mg/L |
|  | Intermittent | 0,36 mg/L | Sediment (Fresh water) | 0,981 mg/kg |
|  | Oral | Non-applicable | Sediment (Marine water) | 0,098 mg/kg |

## SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION (continued)

| Identification |  |  |  |  |
| --- | --- | --- | --- | --- |
| heptan-2-one CAS: 110-43-0 EC: 203-767-1 | STP | 12,5 mg/L | Fresh water | 0,098 mg/L |
|  | Soil | 0,321 mg/kg | Marine water | 0,01 mg/L |
|  | Intermittent | 0,982 mg/L | Sediment (Fresh water) | 1,89 mg/kg |
|  | Oral | Non-applicable | Sediment (Marine water) | 0,189 mg/kg |
| Xylene CAS: 1330-20-7 EC: 215-535-7 | STP | 6,58 mg/L | Fresh water | 0,327 mg/L |
|  | Soil | 2,31 mg/kg | Marine water | 0,327 mg/L |
|  | Intermittent | 0,327 mg/L | Sediment (Fresh water) | 12,46 mg/kg |
|  | Oral | Non-applicable | Sediment (Marine water) | 12,46 mg/kg |

  - Exposure controls:

A.- Individual protection measures, such as personal protective equipment

As a preventative measure it is recommended to use basic Personal Protective Equipment, with the corresponding <<CE marking>> in accordance with Regulation (EU) 2016/425. For more information on Personal Protective Equipment (storage, use, cleaning, maintenance, class of protection,…) consult the information leaflet provided by the manufacturer. For more information see subsection 7.1. All information contained herein is a recommendation which needs some specification from the labour risk prevention services as it is not known whether the company has additional measures at its disposal.

B.- Respiratory protection

| Pictogram | PPE | Labelling | CEN Standard | Remarks |
| --- | --- | --- | --- | --- |
| Mandatory respiratory tract protection | Filter mask for gases and vapours |  | EN 405:2002+A1:2010 | Replace when there is a taste or smell of the contaminant inside the face mask. If the contaminant comes with warnings it is recommended to use isolation equipment. |

C.- Specific protection for the hands

| Pictogram | PPE | Labelling | CEN Standard | Remarks |
| --- | --- | --- | --- | --- |
| Mandatory hand protection | Chemical protective gloves (Material: Linear low-density polyethylene (LLDPE), Breakthrough time: > 480 min, Thickness: 0.062 mm) |  | EN ISO 21420:2020 | Replace the gloves at any sign of deterioration. |

As the product is a mixture of several substances, the resistance of the glove material can not be calculated in advance with total reliability and has therefore to be checked prior to the application.

D.- Eye and face protection

| Pictogram | PPE | Labelling | CEN Standard | Remarks |
| --- | --- | --- | --- | --- |
| Mandatory face | Panoramic glasses against splash/projections. |  | EN 166:2002 EN ISO 4007:2018 | Clean daily and disinfect periodically according to the manufacturer´s instructions. Use if there is a risk of splashing. |
| protection |  |  |  |  |

E.- Body protection

| Pictogram | PPE | Labelling | CEN Standard | Remarks |
| --- | --- | --- | --- | --- |
| Mandatory complete body protection | Antistatic and fireproof protective clothing |  | EN 1149-1:2006 EN 1149-2:1997 EN 1149-3:2004 EN 168:2002 EN ISO 14116:2015 EN 1149-5:2018 | Limited protection against flames. |
| Mandatory foot protection | Safety footwear with antistatic and heat resistant properties |  | EN ISO 13287:2020 EN ISO 20345:2011 | Replace boots at any sign of deterioration. |

F.- Additional emergency measures

Environmental exposure controls:

In accordance with the community legislation for the protection of the environment it is recommended to avoid environmental spillage of both the product and its container. For additional information see subsection 7.1.D

Volatile organic compounds:

With regard to Directive 2010/75/EU, this product has the following characteristics:

V.O.C. (Supply):	43,21 % weight

V.O.C. density at 20 ºC:	439,42 kg/m³ (439,42 g/L) Average carbon number:	7,43

Average molecular weight:	115,71 g/mol

SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION (continued)

*Not relevant due to the nature of the product, not providing information property of its hazards.

138 ºC

675 Pa

3611,42 Pa (3,61 kPa)

Non-applicable *

1017 kg/m³

Non-applicable * 3000 cP

3014,17 mm²/s Non-applicable * Non-applicable * Non-applicable * Non-applicable * Non-applicable * Non-applicable * Non-applicable * Non-applicable * Non-applicable *

35 ºC

Liquid Fluid Colourless

Characteristic

Non-applicable *

Physical state at 20 ºC: Appearance:

Colour:

Odour:

Odour threshold:

Volatility:

Boiling point at atmospheric pressure: Vapour pressure at 20 ºC:

Vapour pressure at 50 ºC: Evaporation rate at 20 ºC: Product description:

Density at 20 ºC:

Relative density at 20 ºC:

Dynamic viscosity at 20 ºC: Kinematic viscosity at 20 ºC: Kinematic viscosity at 40 ºC: Concentration:

pH:

Vapour density at 20 ºC:

Partition coefficient n-octanol/water 20 ºC: Solubility in water at 20 ºC:

Solubility properties: Decomposition temperature: Melting point/freezing point: Flammability:

Flash Point:

9.1	Information on basic physical and chemical properties:

For complete information see the product datasheet.

Appearance:

SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES

| Emergency measure | Standards | Emergency measure | Standards |
| --- | --- | --- | --- |
|  | ANSI Z358-1 ISO 3864-1:2011, ISO 3864-4:2011 |  | DIN 12 899 ISO 3864-1:2011, ISO 3864-4:2011 |
| Emergency shower |  | Eyewash stations |  |

*Not relevant due to the nature of the product, not providing information property of its hazards.

Non-applicable *

Non-applicable *

Non-applicable * Non-applicable * Non-applicable * Non-applicable *

Non-applicable *

Explosive properties:

Oxidising properties:

Corrosive to metals:

Heat of combustion:

Aerosols-total percentage (by mass) of flammable components:

Other safety characteristics:

Surface tension at 20 ºC: Refraction index:

Information with regard to physical hazard classes:

9.2

Non-applicable * 180 ºC

Not available Not available

Non-applicable

Flammability (solid, gas): Autoignition temperature: Lower flammability limit: Upper flammability limit: Particle characteristics: Median equivalent diameter:

Other information:

SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES (continued)

  - Reactivity:

No hazardous reactions are expected because the product is stable under recommended storage conditions. See section 7.

  - Chemical stability:

Chemically stable under the indicated conditions of storage, handling and use.

  - Possibility of hazardous reactions:

Under the specified conditions, hazardous reactions that lead to excessive temperatures or pressure are not expected.

  - Conditions to avoid:

Applicable for handling and storage at room temperature:

10.5 Incompatible materials:

10.6 Hazardous decomposition products:

See subsection 10.3, 10.4 and 10.5 to find out the specific decomposition products. Depending on the decomposition conditions, complex mixtures of chemical substances can be released: carbon dioxide (CO₂), carbon monoxide and other organic compounds.

SECTION 10: STABILITY AND REACTIVITY

11.1 Information on hazard classes as defined in Regulation (EC) No 1272/2008:

The experimental information related to the toxicological properties of the product itself is not available

Dangerous health implications:

In case of exposure that is repetitive, prolonged or at concentrations higher than the recommended occupational exposure limits, adverse effects on health may result, depending on the means of exposure:

A- Ingestion (acute effect):

SECTION 11: TOXICOLOGICAL INFORMATION

| Shock and friction | Contact with air | Increase in temperature | Sunlight | Humidity |
| --- | --- | --- | --- | --- |
| Not applicable | Not applicable | Risk of combustion | Avoid direct impact | Not applicable |

| Acids | Water | Oxidising materials | Combustible materials | Others |
| --- | --- | --- | --- | --- |
| Avoid strong acids | Not applicable | Avoid direct impact | Not applicable | Avoid alkalis or strong bases |

## SECTION 11: TOXICOLOGICAL INFORMATION (continued)

    - Acute toxicity : Based on available data, the classification criteria are not met, however, it contains substances classified as dangerous for consumption. For more information see section 3.

    - Corrosivity/Irritability: The consumption of a considerable dose can cause irritation in the throat, abdominal pain, nausea and vomiting.

B- Inhalation (acute effect):

    - Acute toxicity : Exposure in high concentration can interfere with the central nervous system causing headache, dizziness, vertigo, nausea, vomiting, confusion, and in serious cases, loss of consciousness.

    - Corrosivity/Irritability: Causes irritation in respiratory passages, which is normally reversible and limited to the upper respiratory passages.

C- Contact with the skin and the eyes (acute effect):

    - Contact with the skin: Produces skin inflammation.

    - Contact with the eyes: Produces eye damage after contact.

D- CMR effects (carcinogenicity, mutagenicity and toxicity to reproduction):

    - Carcinogenicity: Based on available data, the classification criteria are not met, as it does not contain substances classified as hazardous for the effects mentioned. For more information see section 3.

IARC: Xylene (3); Hydrocarbons, C9, aromatics (3)

    - Mutagenicity: Based on available data, the classification criteria are not met, as it does not contain substances classified as hazardous for this effect. For more information see section 3.

    - Reproductive toxicity: Based on available data, the classification criteria are not met, as it does not contain substances classified as hazardous for this effect. For more information see section 3.

E- Sensitizing effects:

    - Respiratory: Based on available data, the classification criteria are not met, as it does not contain substances classified as hazardous with sensitising effects. For more information see section 3.

    - Skin: Prolonged contact with the skin can result in episodes of allergic contact dermatitis.

F- Specific target organ toxicity (STOT) - single exposure:

Causes irritation in respiratory passages, which is normally reversible and limited to the upper respiratory passages.

G- Specific target organ toxicity (STOT)-repeated exposure:

    - Specific target organ toxicity (STOT)-repeated exposure: Exposure in high concentration can interfere with the central nervous system causing headache, dizziness, vertigo, nausea, vomiting, confusion, and in serious cases, loss of consciousness.

    - Skin: Based on available data, the classification criteria are not met. However, it does contain substances which are classified as dangerous due to repetitive exposure. For more information see section 3.

H- Aspiration hazard:

Based on available data, the classification criteria are not met. However, it does contain substances classified as hazardous for this effect. For more information see section 3.

Other information:

Non-applicable

Specific toxicology information on the substances:

| Identification | Acute toxicity |  | Genus |
| --- | --- | --- | --- |
| Hexamethylene diisocyanate, oligomers CAS: 28182-81-2 EC: 931-274-8 | LD50 oral | 5100 mg/kg | Rat |
|  | LD50 dermal | >2000 mg/kg |  |
|  | LC50 inhalation | 11 mg/L (ATEi) |  |
| N-butyl acetate CAS: 123-86-4 EC: 204-658-1 | LD50 oral | 12789 mg/kg | Rat |
|  | LD50 dermal | 14112 mg/kg | Rabbit |
|  | LC50 inhalation | 23,4 mg/L (4 h) | Rat |
| heptan-2-one CAS: 110-43-0 EC: 203-767-1 | LD50 oral | 1600 mg/kg | Rat |
|  | LD50 dermal | >2000 mg/kg |  |
|  | LC50 inhalation | 11 mg/L (4 h) | Rat |
| Xylene CAS: 1330-20-7 EC: 215-535-7 | LD50 oral | 2100 mg/kg | Rat |
|  | LD50 dermal | 1100 mg/kg | Rat |
|  | LC50 inhalation | 11 mg/L (ATEi) |  |

Acute Toxicity Estimate (ATE mix):

11.2 Information on other hazards:

Endocrine disrupting properties

Endocrine-disrupting properties: The product fails to meet the criteria.

Other information

Non-applicable

SECTION 11: TOXICOLOGICAL INFORMATION (continued)

The experimental information related to the eco-toxicological properties of the product itself is not available

12.1 Toxicity:

Acute toxicity:

Chronic toxicity:

12.2 Persistence and degradability:

Substance-specific information:

SECTION 12: ECOLOGICAL INFORMATION

| Identification | Acute toxicity |  | Genus |
| --- | --- | --- | --- |
| Hydrocarbons, C9, aromatics CAS: 128601-23-0 EC: 918-668-5 | LD50 oral | >2000 mg/kg |  |
|  | LD50 dermal | >2000 mg/kg |  |
|  | LC50 inhalation | >20 mg/L |  |

| ATE mix |  | Ingredient(s) of unknown toxicity |
| --- | --- | --- |
| Oral | 16000 mg/kg (Calculation method) | 0 % |
| Dermal | 11000 mg/kg (Calculation method) | 0 % |
| Inhalation | 14,33 mg/L (4 h) (Calculation method) | 0 % |

| Identification | Concentration |  | Species | Genus |
| --- | --- | --- | --- | --- |
| Hexamethylene diisocyanate, oligomers | LC50 | Non-applicable |  |  |
| CAS: 28182-81-2 | EC50 | Non-applicable |  |  |
| EC: 931-274-8 | EC50 | 1000 mg/L (72 h) | Scenedesmus subspicatus | Algae |
| N-butyl acetate | LC50 | Non-applicable |  |  |
| CAS: 123-86-4 | EC50 | Non-applicable |  |  |
| EC: 204-658-1 | EC50 | 675 mg/L (72 h) | Scenedesmus subspicatus | Algae |
| heptan-2-one | LC50 | 131 mg/L (96 h) | Pimephales promelas | Fish |
| CAS: 110-43-0 | EC50 | Non-applicable |  |  |
| EC: 203-767-1 | EC50 | Non-applicable |  |  |
| Xylene | LC50 | >10 - 100 mg/L (96 h) |  | Fish |
| CAS: 1330-20-7 | EC50 | >10 - 100 mg/L (48 h) |  | Crustacean |
| EC: 215-535-7 | EC50 | >10 - 100 mg/L (72 h) |  | Algae |
| Hydrocarbons, C9, aromatics | LC50 | >1 - 10 mg/L (96 h) |  | Fish |
| CAS: 128601-23-0 | EC50 | >1 - 10 mg/L (48 h) |  | Crustacean |
| EC: 918-668-5 | EC50 | >1 - 10 mg/L (72 h) |  | Algae |

| Identification | Concentration |  | Species | Genus |
| --- | --- | --- | --- | --- |
| N-butyl acetate CAS: 123-86-4 EC: 204-658-1 | NOEC | Non-applicable |  |  |
|  | NOEC | 23,2 mg/L | Daphnia magna | Crustacean |
| Xylene CAS: 1330-20-7 EC: 215-535-7 | NOEC | 1,3 mg/L | Oncorhynchus mykiss | Fish |
|  | NOEC | 1,17 mg/L | Ceriodaphnia dubia | Crustacean |

| Identification | Degradability |  | Biodegradability |  |
| --- | --- | --- | --- | --- |
| N-butyl acetate CAS: 123-86-4 EC: 204-658-1 | BOD5 | Non-applicable | Concentration | Non-applicable |
|  | COD | Non-applicable | Period | 5 days |
|  | BOD5/COD | Non-applicable | % Biodegradable | 84 % |

12.3 Bioaccumulative potential:

Substance-specific information:

12.4 Mobility in soil:

  - Results of PBT and vPvB assessment:

Product fails to meet PBT/vPvB criteria

  - Endocrine disrupting properties:

Endocrine-disrupting properties: The product fails to meet the criteria.

  - Other adverse effects:

Not described

SECTION 12: ECOLOGICAL INFORMATION (continued)

13.1 Waste treatment methods:

Type of waste (Regulation (EU) No 1357/2014):

HP14 Ecotoxic, HP3 Flammable, HP5 Specific Target Organ Toxicity (STOT)/Aspiration Toxicity, HP6 Acute Toxicity, HP13 Sensitising, HP4 Irritant — skin irritation and eye damage

Waste management (disposal and evaluation):

Consult the authorized waste service manager on the assessment and disposal operations in accordance with Annex 1 and Annex 2 (Directive 2008/98/EC). As under 15 01 (2014/955/EC) of the code and in case the container has been in direct contact with the product, it will be processed the same way as the actual product. Otherwise, it will be processed as non-dangerous residue. Waste should not be disposed of to drains. See paragraph 6.2.

Regulations related to waste management:

In accordance with Annex II of Regulation (EC) No 1907/2006 (REACH) the community or state provisions related to waste management are stated

SECTION 13: DISPOSAL CONSIDERATIONS

| Identification | Degradability |  | Biodegradability |  |
| --- | --- | --- | --- | --- |
| Xylene CAS: 1330-20-7 EC: 215-535-7 | BOD5 | Non-applicable | Concentration | Non-applicable |
|  | COD | Non-applicable | Period | 28 days |
|  | BOD5/COD | Non-applicable | % Biodegradable | 88 % |

| Identification | Bioaccumulation potential |  |
| --- | --- | --- |
| N-butyl acetate | BCF | 4 |
| CAS: 123-86-4 | Pow Log | 1.78 |
| EC: 204-658-1 | Potential | Low |
| heptan-2-one | BCF | 7 |
| CAS: 110-43-0 | Pow Log | 1.98 |
| EC: 203-767-1 | Potential | Low |
| Xylene | BCF | 9 |
| CAS: 1330-20-7 | Pow Log | 2.77 |
| EC: 215-535-7 | Potential | Low |

| Identification | Absorption/desorption |  | Volatility |  |
| --- | --- | --- | --- | --- |
| N-butyl acetate CAS: 123-86-4 EC: 204-658-1 | Koc | Non-applicable | Henry | Non-applicable |
|  | Conclusion | Non-applicable | Dry soil | Non-applicable |
|  | Surface tension | 2,478E-2 N/m (25 ºC) | Moist soil | Non-applicable |
| heptan-2-one CAS: 110-43-0 EC: 203-767-1 | Koc | 280 | Henry | 17,12 Pa·m³/mol |
|  | Conclusion | Moderate | Dry soil | Yes |
|  | Surface tension | 2,612E-2 N/m (25 ºC) | Moist soil | Yes |
| Xylene CAS: 1330-20-7 EC: 215-535-7 | Koc | 202 | Henry | 524,86 Pa·m³/mol |
|  | Conclusion | Moderate | Dry soil | Yes |
|  | Surface tension | Non-applicable | Moist soil | Yes |

| Code | Description | Waste class (Regulation (EU) No 1357/2014) |
| --- | --- | --- |
| 15 01 10* | packaging containing residues of or contaminated by hazardous substances | Dangerous |

Community legislation: Directive 2008/98/EC, 2014/955/EU, Regulation (EU) No 1357/2014

SECTION 13: DISPOSAL CONSIDERATIONS (continued)

SECTION 14: TRANSPORT INFORMATION

Transport of dangerous goods by land:

With regard to ADR 2021 and RID 2021:

  - UN number or ID number:	UN1263

  - UN proper shipping name:	PAINT

  - Transport hazard class(es):	3

Labels:	3

  - Packing group:	III

  - Environmental hazards:	No

  - Special precautions for user

Special regulations:	163, 367, 650

Tunnel restriction code:	D/E

Physico-Chemical properties:	see section 9 Limited quantities:	5 L

  - Maritime transport in bulk according to IMO instruments:

Transport of dangerous goods by sea:

With regard to IMDG 40-20:

Non-applicable

  - UN number or ID number:	UN1263

  - UN proper shipping name:	PAINT

  - Transport hazard class(es):	3

Labels:	3

  - Packing group:	III

  - Marine pollutant:	No

  - Special precautions for user

Special regulations:	223, 955, 163, 367

EmS Codes:	F-E, S-E

Physico-Chemical properties:	see section 9 Limited quantities:	5 L

Segregation group:	Non-applicable

  - Maritime transport in bulk according to IMO instruments:

Transport of dangerous goods by air:

With regard to IATA/ICAO 2022:

Non-applicable

  - UN number or ID number:	UN1263

  - UN proper shipping name:	PAINT

  - Transport hazard class(es):	3

Labels:	3

  - Packing group:	III

  - Environmental hazards:	No

  - Special precautions for user

Physico-Chemical properties:	see section 9

SECTION 15: REGULATORY INFORMATION

Maritime transport in bulk according to IMO instruments:

Non-applicable

15.1 Safety, health and environmental regulations/legislation specific for the substance or mixture: Candidate substances for authorisation under the Regulation (EC) No 1907/2006 (REACH): Non-applicable Substances included in Annex XIV of REACH ("Authorisation List") and sunset date: Non-applicable Regulation (EC) No 1005/2009, about substances that deplete the ozone layer: Non-applicable

Article 95, REGULATION (EU) No 528/2012: Non-applicable

REGULATION (EU) No 649/2012, in relation to the import and export of hazardous chemical products: Non-applicable

Seveso III:

Limitations to commercialisation and the use of certain dangerous substances and mixtures (Annex XVII REACH, etc ….):

SECTION 15: REGULATORY INFORMATION (continued)

| Section | Description | Lower-tier requirements | Upper-tier requirements |
| --- | --- | --- | --- |
| P5c | FLAMMABLE LIQUIDS | 5000 | 50000 |

## SECTION 15: REGULATORY INFORMATION (continued)

Shall not be used in:

—ornamental articles intended to produce light or colour effects by means of different phases, for example in ornamental lamps and ashtrays,

—tricks and jokes,

—games for one or more participants, or any article intended to be used as such, even with ornamental aspects.

Contains more than 0.1 % of Hexamethylene diisocyanate, oligomers by weight. 1. Shall not be used as substances on their own, as a constituent in other substances or in mixtures for industrial and professional use(s) after 24 August 2023, unless:

(a) the concentration of diisocyanates individually and in combination is less than 0,1 % by weight, or (b) the employer or self- employed ensures that industrial or professional user(s) have successfully completed training on the safe use of diisocyanates prior to the use of the substance(s) or mixture(s).

- Shall not be placed on the market as substances on their own, as a constituent in other substances or in mixtures for industrial and professional use(s) after 24 February 2022, unless:

  - the concentration of diisocyanates individually and in combination is less than 0,1 % by weight, or (b) the supplier ensures that the recipient of the substance(s) or mixture(s) is provided with information on the requirements referred to in point (b) of paragraph 1 and the following statement is placed on the packaging, in a manner that is visibly distinct from the rest of the label information: “As from 24 August 2023 adequate training is required before industrial or professional use”.

- For the purpose of this entry “industrial and professional user(s)” means any worker or self-employed worker handling diisocyanates on their own, as a constituent in other substances or in mixtures for industrial and professional use(s) or supervising these tasks.

- The training referred to in point (b) of paragraph 1 shall include the instructions for the control of dermal and inhalation exposure to diisocyanates at the workplace without prejudice to any national occupational exposure limit value or other appropriate risk management measures at national level. Such training shall be conducted by an expert on occupational safety and health with competence acquired by relevant vocational training. That training shall cover as a minimum:

  - the training elements in point (a) of paragraph 5 for all industrial and professional use(s).

  - the training elements in points (a) and (b) of paragraph 5 for the following uses:

- handling open mixtures at ambient temperature (including foam tunnels)

- spraying in a ventilated booth

- application by roller

- application by brush

- application by dipping and pouring

- mechanical post treatment (e.g. cutting) of not fully cured articles which are not warm anymore

- cleaning and waste

- any other uses with similar exposure through the dermal and/or inhalation route

  - the training elements in points (a), (b) and (c) of paragraph 5 for the following uses:

- handling incompletely cured articles (e.g. freshly cured, still warm)

- foundry applications

- maintenance and repair that needs access to equipment

- open handling of warm or hot formulations (> 45 °C)

- spraying in open air, with limited or only natural ventilation (includes large industry working halls) and spraying with high energy (e.g. foams, elastomers)

- and any other uses with similar exposure through the dermal and/or inhalation route.

- Training elements:

  - general training, including on-line training, on:

- chemistry of diisocyanates

- toxicity hazards (including acute toxicity)

- exposure to diisocyanates

- occupational exposure limit values

- how sensitisation can develop

- odour as indication of hazard

- importance of volatility for risk

- viscosity, temperature, and molecular weight of diisocyanates

- personal hygiene

- personal protective equipment needed, including practical instructions for its correct use and its limitations

- risk of dermal contact and inhalation exposure

- risk in relation to application process used

- skin and inhalation protection scheme

- ventilation

- cleaning, leakages, maintenance

- discarding empty packaging

- protection of bystanders

- identification of critical handling stages

- specific national code systems (if applicable)

- behaviour-based safety

## SECTION 15: REGULATORY INFORMATION (continued)

- certification or documented proof that training has been successfully completed

  - intermediate level training, including on-line training, on:

- additional behaviour-based aspects

- maintenance

- management of change

- evaluation of existing safety instructions

- risk in relation to application process used

- certification or documented proof that training has been successfully completed

  - advanced training, including on-line training, on:

- any additional certification needed for the specific uses covered

- spraying outside a spraying booth

- open handling of hot or warm formulations (> 45 °C)

- certification or documented proof that training has been successfully completed

- The training shall comply with the provisions set by the Member State in which the industrial or professional user(s) operate. Member States may implement or continue to apply their own national requirements for the use of the substance(s) or mixture (s), as long as the minimum requirements set out in paragraphs 4 and 5 are met.

- The supplier referred to in point (b) of paragraph 2 shall ensure that the recipient is provided with training material and courses pursuant to paragraphs 4 and 5 in the official language(s) of the Member State(s) where the substance(s) or mixture(s) are supplied. The training shall take into consideration the specificity of the products supplied, including composition, packaging, and design.

- The employer or self-employed shall document the successful completion of the training referred to in paragraphs 4 and 5. The training shall be renewed at least every five years.

- Member States shall include in their reports pursuant to Article 117(1) the following information:

  - any established training requirements and other risk management measures related to the industrial and professional uses of diisocyanates foreseen in national law

  - the number of cases of reported and recognised occupational asthma and occupational respiratory and dermal diseases in relation to diisocyanates

  - national exposure limits for diisocyanates, if there are any

  - information about enforcement activities related to this restriction.

- This restriction shall apply without prejudice to other Union legislation on the protection of safety and health of workers at the workplace.

Specific provisions in terms of protecting people or the environment:

It is recommended to use the information included in this safety data sheet as a basis for conducting workplace-specific risk assessments in order to establish the necessary risk prevention measures for the handling, use, storage and disposal of this product.

Other legislation:

The product could be affected by sectorial legislation

15.2 Chemical safety assessment:

Legislation related to safety data sheets:

The SDS shall be supplied in an official language of the country where the product is placed on the market. This safety data sheet has been designed in accordance with ANNEX II-Guide to the compilation of safety data sheets of Regulation (EC) No 1907/2006 (COMMISSION REGULATION (EU) 2020/878).

Modifications related to the previous Safety Data Sheet which concerns the ways of managing risks.:

COMMISSION REGULATION (EU) 2020/878

Texts of the legislative phrases mentioned in section 2:

H317: May cause an allergic skin reaction. H335: May cause respiratory irritation.

H336: May cause drowsiness or dizziness. H315: Causes skin irritation.

H412: Harmful to aquatic life with long lasting effects.

H373: May cause damage to organs through prolonged or repeated exposure (Oral). H332: Harmful if inhaled.

H226: Flammable liquid and vapour. H319: Causes serious eye irritation.

Texts of the legislative phrases mentioned in section 3:

SECTION 16: OTHER INFORMATION

The supplier has not carried out evaluation of chemical safety.

## SECTION 16: OTHER INFORMATION (continued)

The phrases indicated do not refer to the product itself; they are present merely for informative purposes and refer to the individual components which appear in section 3

CLP Regulation (EC) No 1272/2008:

Acute Tox. 4: H302+H332 - Harmful if swallowed or if inhaled.

Acute Tox. 4: H312+H332 - Harmful in contact with skin or if inhaled. Acute Tox. 4: H332 - Harmful if inhaled.

Aquatic Chronic 2: H411 - Toxic to aquatic life with long lasting effects. Aquatic Chronic 3: H412 - Harmful to aquatic life with long lasting effects. Asp. Tox. 1: H304 - May be fatal if swallowed and enters airways.

Eye Irrit. 2: H319 - Causes serious eye irritation. Flam. Liq. 3: H226 - Flammable liquid and vapour. Skin Irrit. 2: H315 - Causes skin irritation.

Skin Sens. 1: H317 - May cause an allergic skin reaction.

STOT RE 2: H373 - May cause damage to organs through prolonged or repeated exposure (Oral). STOT SE 3: H335 - May cause respiratory irritation.

STOT SE 3: H336 - May cause drowsiness or dizziness.

Classification procedure:

Skin Sens. 1: Calculation method STOT SE 3: Calculation method STOT SE 3: Calculation method Skin Irrit. 2: Calculation method

Aquatic Chronic 3: Calculation method STOT RE 2: Calculation method

Acute Tox. 4: Calculation method

Flam. Liq. 3: Calculation method (2.6.4.3) Eye Irrit. 2: Calculation method

Advice related to training:

Training is recommended in order to prevent industrial risks for staff using this product and to facilitate their comprehension and interpretation of this safety data sheet, as well as the label on the product.

Principal bibliographical sources:

http://echa.europa.eu http://eur-lex.europa.eu

Abbreviations and acronyms:

ADR: European agreement concerning the international carriage of dangerous goods by road IMDG: International maritime dangerous goods code

IATA: International Air Transport Association ICAO: International Civil Aviation Organisation COD: Chemical Oxygen Demand

BOD5: 5day biochemical oxygen demand BCF: Bioconcentration factor

LD50: Lethal Dose 50

LC50: Lethal Concentration 50 EC50: Effective concentration 50

LogPOW: Octanolwater partition coefficient Koc: Partition coefficient of organic carbon UFI: unique formula identifier

IARC: International Agency for Research on Cancer

The information contained in this safety data sheet is based on sources, technical knowledge and current legislation at European and state level, without being able to guarantee its accuracy. This information cannot be considered a guarantee of the properties of the product, it is simply a description of the security requirements. The occupational methodology and conditions for users of this product are not within our awareness or control, and it is ultimately the responsibility of the user to take the necessary measures to obtain the legal requirements concerning the manipulation, storage, use and disposal of chemical products. The information on this safety data sheet only refers to this product, which should not be used for needs other than those specified.

- END OF SAFETY DATA SHEET -