  - Relevant identified uses of the substance or mixture and uses advised against:

Relevant uses: Car repair; paints and varnishes. For professional users only. Uses advised against: All uses not specified in this section or in section 7.3

  - Details of the supplier of the safety data sheet:

Rags Ltd.

Durbes str. 8, Riga, LV 1007

Latvija

www.rags.lv

+37167808780

rags@rags.lv

1.4	Emergency telephone number: +371 67042473.; 112

STONDER Starline VHS Crystal Clearcoat

KAE7-M7D3-H00W-1MJ0

1.1	Product identifier:

Other means of identification: UFI:

SECTION 1: IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

# SECTION 2: HAZARDS IDENTIFICATION **

## Classification of the substance or mixture: CLP Regulation (EC) No 1272/2008:

Classification of this product has been carried out in accordance with CLP Regulation (EC) No 1272/2008.

Aquatic Chronic 3: Hazardous to the aquatic environment, long-term hazard, Category 3, H412 Carc. 2: Carcinogenicity, Category 2, H351

Eye Irrit. 2: Eye irritation, Category 2, H319

Flam. Liq. 3: Flammable liquids, Category 3, H226 Muta. 2: Germ cell mutagenicity, Category 2, H341 Repr. 1B: Reproductive toxicity, Category 1B, H360 Skin Sens. 1A: Sensitisation, skin, Category 1A, H317

STOT RE 2: Specific target organ toxicity — Repeated exposure, Hazard Category 2 (Oral), H373 STOT SE 2: Specific target organ toxicity — single exposure, Hazard Category 2, H371

STOT SE 3: Specific toxicity causing drowsiness and dizziness, single exposure, Category 3, H336

## Label elements:

CLP Regulation (EC) No 1272/2008:

Danger

## Hazard statements:

Aquatic Chronic 3: H412 - Harmful to aquatic life with long lasting effects. Carc. 2: H351 - Suspected of causing cancer.

Eye Irrit. 2: H319 - Causes serious eye irritation. Flam. Liq. 3: H226 - Flammable liquid and vapour. Muta. 2: H341 - Suspected of causing genetic defects.

Repr. 1B: H360 - May damage fertility or the unborn child. Skin Sens. 1A: H317 - May cause an allergic skin reaction.

STOT RE 2: H373 - May cause damage to organs through prolonged or repeated exposure (Oral). STOT SE 2: H371 - May cause damage to organs.

STOT SE 3: H336 - May cause drowsiness or dizziness.

Precautionary statements:

P210: Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. P280: Wear protective gloves/face protection/protective clothing/respiratory protection/protective footwear. P302+P352: IF ON SKIN: Wash with plenty of water.

P304+P340: IF INHALED: Remove person to fresh air and keep comfortable for breathing.

P305+P351+P338: IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

P308+P313: IF exposed or concerned: Get medical advice/attention.

P501: Dispose of contents/container in accordance with regulations on hazardous waste or packaging and packaging waste respectively.

Supplementary information:

EUH066: Repeated exposure may cause skin dryness or cracking.

EUH208: Contains Hydroxyphenyl benzotriazol derivative, Reaction mass of Bis(1,2,2,6,6-pentamethyl-4-piperidyl) sebacate and Methyl 1,2,2,6,6-pentamethyl-4-piperidyl sebacate. May produce an allergic reaction.

Substances that contribute to the classification

N-butyl acetate; 4-methylpentan-2-one; Hydrocarbons, C9, aromatics; Dibutyltin Dilaurate

Additional Labelling:

Restricted to professional users

2.3	Other hazards:

Product does not meet PBT/vPvB criteria

Endocrine-disrupting properties: The product does not meet the criteria.

SECTION 2: HAZARDS IDENTIFICATION  (continued)

| Identification |  | Chemical name/Classification |  |  | Concentration |
| --- | --- | --- | --- | --- | --- |
| CAS: | 123-86-4 | N-butyl acetate⁽¹⁾ |  | ATP CLP00 |  |
| EC: Index: REACH: | 204-658-1 607-025-00-1 01-2119485493-29- |  |  |  | 10 - <25 % |
|  |  | Regulation 1272/2008 | Flam. Liq. 3: H226; STOT SE 3: H336; EUH066 - Warning |  |  |
|  | XXXX |  |  |  |  |
| CAS: | 110-43-0 | heptan-2-one⁽¹⁾ |  | ATP CLP00 |  |
| EC: Index: REACH: | 203-767-1 606-024-00-3 01-2119902391-49- |  |  |  | 5 - <10 % |
|  |  | Regulation 1272/2008 | Acute Tox. 4: H302+H332; Flam. Liq. 3: H226 - Warning |  |  |
|  | XXXX |  |  |  |  |
| CAS: | 108-10-1 | 4-methylpentan-2-one⁽¹⁾ |  | ATP ATP17 |  |
| EC: Index: REACH: | 203-550-1 606-004-00-4 01-2119473980-30- |  |  |  |  |
|  |  | Regulation 1272/2008 | Acute Tox. 4: H332; Carc. 2: H351; Eye Irrit. 2: H319; Flam. Liq. 2: H225; STOT SE 3: H336; EUH066 - Danger |  | 5 - <10 % |
|  | XXXX |  |  |  |  |
| CAS: | 1330-20-7 | Xylene⁽¹⁾ |  | Self-classified |  |
| EC: Index: REACH: | 215-535-7 601-022-00-9 01-2119488216-32- XXXX |  |  |  | 2,5 - <5 % |
|  |  | Regulation 1272/2008 | Acute Tox. 4: H312+H332; Aquatic Chronic 3: H412; Asp. Tox. 1: H304; Eye Irrit. 2: H319; Flam. Liq. 3: H226; Skin Irrit. 2: H315; STOT RE 2: H373; STOT SE 3: H335 - Danger |  |  |
| CAS: | 128601-23-0 | Hydrocarbons, C9, aromatics⁽¹⁾ |  | Self-classified |  |
| EC: Index: REACH: | 918-668-5 Non-applicable 01-2119455851-35- |  |  |  |  |
|  |  | Regulation 1272/2008 | Aquatic Chronic 2: H411; Asp. Tox. 1: H304; Flam. Liq. 3: H226; STOT SE 3: H335; STOT SE 3: H336; EUH066 - Danger |  | 1 - <2,5 % |
|  | XXXX |  |  |  |  |

  - Substance:

Non-applicable

  - Mixture:

Chemical description: Mixture composed of chemical products

Components:

In accordance with Annex II of Regulation (EC) No 1907/2006 (point 3), the product contains:

⁽¹⁾ Substances presenting a health or environmental hazard which meet criteria laid down in Regulation (EU) No. 2020/878

⁽²⁾ Substance with a Union workplace exposure limit

SECTION 3: COMPOSITION/INFORMATION ON INGREDIENTS

| Identification |  | Chemical name/Classification |  |  | Concentration |
| --- | --- | --- | --- | --- | --- |
| CAS: | 77-58-7 | Dibutyltin Dilaurate⁽¹⁾ |  | Self-classified |  |
| EC: Index: REACH: | 201-039-8 050-030-00-3 01-2119496068-27- |  |  |  |  |
|  |  | Regulation 1272/2008 | Aquatic Acute 1: H400; Aquatic Chronic 1: H410; Eye Irrit. 2: H319; Muta. 2: H341; Repr. 1B: H360; Skin Sens. 1: H317; STOT RE 1: H372; STOT SE 1: H370 - Danger |  | 1 - <2,5 % |
|  | XXXX |  |  |  |  |
| CAS: | 1065336-91-5 | Reaction mass of Bis(1,2,2,6,6-pentamethyl-4-piperidyl) sebacate and Methyl |  | Self-classified |  |
| EC: Index: REACH: | 915-687-0 Non-applicable 01-2119491304-40- XXXX | 1,2,2,6,6-pentamethyl-4-piperidyl sebacate⁽¹⁾ |  |  |  |
|  |  | Regulation 1272/2008 | Aquatic Acute 1: H400; Aquatic Chronic 1: H410; Repr. 2: H361f; Skin Sens. 1A: H317 - Warning |  | <1 % |
| CAS: | Non-applicable | Hydroxyphenyl benzotriazol derivative⁽¹⁾ |  | ATP CLP00 |  |
| EC: Index: REACH: | 400-830-7 607-176-00-3 01-0000015075-76- |  |  |  | <1 % |
|  |  | Regulation 1272/2008 | Aquatic Chronic 2: H411; Skin Sens. 1: H317 - Warning |  |  |
|  | XXXX |  |  |  |  |
| CAS: | 100-41-4 | Ethylbenzene⁽²⁾ |  | ATP ATP06 |  |
| EC: Index: REACH: | 202-849-4 601-023-00-4 01-2119489370-35- |  |  |  |  |
|  |  | Regulation 1272/2008 | Acute Tox. 4: H332; Asp. Tox. 1: H304; Flam. Liq. 2: H225; STOT RE 2: H373 - Danger |  | <1 % |
|  | XXXX |  |  |  |  |
| CAS: | 108-88-3 | Toluene⁽²⁾ |  | Self-classified |  |
| EC: Index: REACH: | 203-625-9 601-021-00-3 01-2119471310-51- |  |  |  |  |
|  |  | Regulation 1272/2008 | Aquatic Chronic 3: H412; Asp. Tox. 1: H304; Flam. Liq. 2: H225; Repr. 2: H361d; Skin Irrit. 2: H315; STOT RE 2: H373; STOT SE 3: H336 - Danger |  | <1 % |
|  | XXXX |  |  |  |  |

| Identification | Acute toxicity |  | Genus |
| --- | --- | --- | --- |
| Xylene CAS: 1330-20-7 EC: 215-535-7 | LD50 oral | Non-applicable |  |
|  | LD50 dermal | Non-applicable |  |
|  | LC50 inhalation | 11 mg/L (ATEi) |  |
| 4-methylpentan-2-one CAS: 108-10-1 EC: 203-550-1 | LD50 oral | Non-applicable |  |
|  | LD50 dermal | Non-applicable |  |
|  | LC50 inhalation | 11 mg/L (4 h) (ATEi) | Rat |

⁽¹⁾ Substances presenting a health or environmental hazard which meet criteria laid down in Regulation (EU) No. 2020/878

⁽²⁾ Substance with a Union workplace exposure limit

To obtain more information on the hazards of the substances consult sections 11, 12 and 16.

Acute toxicity estimate for the substance in Part 3 of Annex VI to Regulation (EC) No 1272/2008 or as determined in accordance with Annex I to that Regulation:

SECTION 3: COMPOSITION/INFORMATION ON INGREDIENTS  (continued)

4.1	Description of first aid measures:

The symptoms resulting from intoxication can appear after exposure, therefore, in case of doubt, seek medical attention for direct exposure to the chemical product or persistent discomfort, showing the SDS of this product.

By inhalation:

Remove the person affected from the area of exposure, provide with fresh air and keep at rest. In serious cases such as cardiorespiratory failure, artificial resuscitation techniques will be necessary (mouth to mouth resuscitation, cardiac massage, oxygen supply, etc.) requiring immediate medical assistance.

By skin contact:

Remove contaminated clothing and footwear, rinse skin or shower the person affected if appropriate with plenty of cold water and neutral soap. In serious cases see a doctor. If the product causes burns or freezing, clothing should not be removed as this could worsen the injury caused if it is stuck to the skin. If blisters form on the skin, these should never be burst as this will increase the risk of infection.

By eye contact:

Rinse eyes thoroughly with lukewarm water for at least 15 minutes. Do not allow the person affected to rub or close their eyes. If the injured person uses contact lenses, these should be removed unless they are stuck to the eyes, in which case this could cause further damage. In all cases, after cleaning, a doctor should be consulted as quickly as possible with the SDS of the product.

By ingestion/aspiration:

Request medical assistance immediately, showing the SDS of this product. Induce vomiting (ONLY IF PERSON IS CONSCIOUS!) and then ingest large quantities of liquid to dilute the toxin. Keep the person affected at rest.

SECTION 4: FIRST AID MEASURES

7.1	Precautions for safe handling:

A.- General precautions for safe use

SECTION 7: HANDLING AND STORAGE

  - Personal precautions, protective equipment and emergency procedures: For non-emergency personnel:

Isolate leaks provided that there is no additional risk for the people performing this task. Evacuate the area and keep out those without protection. Personal protection equipment must be used against potential contact with the spilt product (See section 8). Above all prevent the formation of any vapour-air flammable mixtures, through either ventilation or the use of an inert medium. Remove any source of ignition. Eliminate electrostatic charges by interconnecting all the conductive surfaces on which static electricity could form, and also ensuring that all surfaces are connected to the ground.

For emergency responders:

Wear protective equipment. Keep unprotected persons away. See section 8.

  - Environmental precautions:

Avoid at all cost any type of spillage into an aqueous medium. Contain the product absorbed appropriately in hermetically sealed containers. Notify the relevant authority in case of exposure to the general public or the environment.

  - Methods and material for containment and cleaning up:

It is recommended:

Absorb the spillage using sand or inert absorbent and move it to a safe place. Do not absorb in sawdust or other combustible absorbents. For any concern related to disposal consult section 13.

  - Reference to other sections:

See sections 8 and 13.

SECTION 6: ACCIDENTAL RELEASE MEASURES

  - Extinguishing media: Suitable extinguishing media:

If possible use polyvalent powder fire extinguishers (ABC powder), alternatively use foam or carbon dioxide extinguishers (CO₂).

Unsuitable extinguishing media:

IT IS RECOMMENDED NOT to use full jet water as an extinguishing agent.

  - Special hazards arising from the substance or mixture:

As a result of combustion or thermal decomposition reactive sub-products are created that can become highly toxic and, consequently, can present a serious health risk.

  - Advice for firefighters:

Depending on the magnitude of the fire it may be necessary to use full protective clothing and self-contained breathing apparatus (SCBA). Minimum emergency facilities and equipment should be available (fire blankets, portable first aid kit,...) in accordance with Directive 89/654/EC.

Additional provisions:

Act in accordance with the Internal Emergency Plan and the Information Sheets on actions to take after an accident or other emergencies. Eliminate all sources of ignition. In case of fire, cool the storage containers and tanks for products susceptible to combustion, explosion or BLEVE as a result of high temperatures. Avoid spillage of the products used to extinguish the fire into an aqueous medium.

SECTION 5: FIREFIGHTING MEASURES

  - Most important symptoms and effects, both acute and delayed:

Acute and delayed effects are indicated in sections 2 and 11.

  - Indication of any immediate medical attention and special treatment needed:

Non-applicable

SECTION 4: FIRST AID MEASURES (continued)

# SECTION 7: HANDLING AND STORAGE (continued)

Comply with the current legislation concerning the prevention of industrial risks. Keep containers hermetically sealed. Control spills and residues, destroying them with safe methods (section 6). Avoid leakages from the container. Maintain order and cleanliness where dangerous products are used.

B.- Technical recommendations for the prevention of fires and explosions

Transfer in well ventilated areas, preferably through localized extraction. Fully control sources of ignition (mobile phones, sparks,…) and ventilate during cleaning operations. Avoid the existence of dangerous atmospheres inside containers, applying inertization systems where possible. Transfer at a slow speed to avoid the creation of electrostatic charges. Against the possibility of electrostatic charges: ensure a perfect equipotential connection, always use groundings, do not wear work clothes made of acrylic fibres, preferably wearing cotton clothing and conductive footwear. Comply with the essential security requirements for equipment and systems defined in Directive 2014/34/EC (ATEX 100) and with the minimum requirements for protecting the security and health of workers under the selection criteria of Directive 1999/92/EC (ATEX 137). Consult section 10 for conditions and materials that should be avoided.

C.- Technical recommendations on general occupational hygiene

PREGNANT WOMEN SHOULD NOT BE EXPOSED TO THIS PRODUCT. Transfer in designated areas that comply with the necessary safety conditions (emergency showers and eyewash stations in close proximity), using personal protection equipment, especially on the hands and face (See section 8). Limit manual transfers to small amounts only. Do not eat or drink during the process, washing hands afterwards with suitable cleaning products.

D.- Technical recommendations to prevent environmental risks

Due to the danger of this product for the environment it is recommended to use it within an area containing contamination control barriers in case of spillage, as well as having absorbent material in close proximity.

## Conditions for safe storage, including any incompatibilities:

A.- Technical measures for storage Minimum Temp.:	15 ºC

Maximum Temp.:	25 ºC

Maximum time:	12 Months B.- General conditions for storage

Avoid sources of heat, radiation, static electricity and contact with food. For additional information see subsection 10.5

## Specific end use(s):

8.1	Control parameters:

Substances whose occupational exposure limits have to be monitored in the workplace (European OEL, not country-specific legislation):

Directive (EU) 2000/39, Directive 2004/37/EC,Directive (EU) 2006/15, Directive (EU) 2009/161, Directive (EU) 2017/164, Directive (EU) 2019/1831:

DNEL (Workers):

SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION

Except for the instructions already specified it is not necessary to provide any special recommendation regarding the uses of this product.

| Identification |  | Occupational exposure limits |  |  |
| --- | --- | --- | --- | --- |
| N-butyl acetate CAS: 123-86-4 | EC: 204-658-1 | IOELV (8h) | 50 ppm | 241 mg/m³ |
|  |  | IOELV (STEL) | 150 ppm | 723 mg/m³ |
| heptan-2-one CAS: 110-43-0 | EC: 203-767-1 | IOELV (8h) | 50 ppm | 238 mg/m³ |
|  |  | IOELV (STEL) | 100 ppm | 475 mg/m³ |
| 4-methylpentan-2-one CAS: 108-10-1 EC: 203-550-1 |  | IOELV (8h) | 20 ppm | 83 mg/m³ |
|  |  | IOELV (STEL) | 50 ppm | 208 mg/m³ |
| Xylene CAS: 1330-20-7 | EC: 215-535-7 | IOELV (8h) | 50 ppm | 221 mg/m³ |
|  |  | IOELV (STEL) | 100 ppm | 442 mg/m³ |
| Ethylbenzene CAS: 100-41-4 | EC: 202-849-4 | IOELV (8h) | 100 ppm | 442 mg/m³ |
|  |  | IOELV (STEL) | 200 ppm | 884 mg/m³ |
| Toluene CAS: 108-88-3 | EC: 203-625-9 | IOELV (8h) | 50 ppm | 192 mg/m³ |
|  |  | IOELV (STEL) | 100 ppm | 384 mg/m³ |

# SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION (continued)

|  |  | Short exposure |  | Long exposure |  |
| --- | --- | --- | --- | --- | --- |
| Identification |  | Systemic | Local | Systemic | Local |
| N-butyl acetate CAS: 123-86-4 EC: 204-658-1 | Oral | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Dermal | 11 mg/kg | Non-applicable | 11 mg/kg | Non-applicable |
|  | Inhalation | 600 mg/m³ | 600 mg/m³ | 300 mg/m³ | 300 mg/m³ |
| heptan-2-one CAS: 110-43-0 EC: 203-767-1 | Oral | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 54,27 mg/kg | Non-applicable |
|  | Inhalation | 1516 mg/m³ | Non-applicable | 394,25 mg/m³ | Non-applicable |
| 4-methylpentan-2-one CAS: 108-10-1 EC: 203-550-1 | Oral | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 11,8 mg/kg | Non-applicable |
|  | Inhalation | 208 mg/m³ | 208 mg/m³ | 83 mg/m³ | 83 mg/m³ |
| Xylene CAS: 1330-20-7 EC: 215-535-7 | Oral | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 212 mg/kg | Non-applicable |
|  | Inhalation | 442 mg/m³ | 442 mg/m³ | 221 mg/m³ | 221 mg/m³ |
| Hydrocarbons, C9, aromatics CAS: 128601-23-0 EC: 918-668-5 | Oral | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 25 mg/kg | Non-applicable |
|  | Inhalation | Non-applicable | Non-applicable | 150 mg/m³ | Non-applicable |
| Dibutyltin Dilaurate CAS: 77-58-7 EC: 201-039-8 | Oral | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Dermal | 2,08 mg/kg | Non-applicable | 0,43 mg/kg | Non-applicable |
|  | Inhalation | 0,059 mg/m³ | Non-applicable | 0,02 mg/m³ | Non-applicable |
| Reaction mass of Bis(1,2,2,6,6-pentamethyl-4-piperidyl) sebacate and Methyl 1,2,2,6,6-pentamethyl-4-piperidyl sebacate CAS: 1065336-91-5 EC: 915-687-0 | Oral | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 0,5 mg/kg | Non-applicable |
|  | Inhalation | Non-applicable | Non-applicable | 0,68 mg/m³ | Non-applicable |
| Hydroxyphenyl benzotriazol derivative CAS: Non-applicable EC: 400-830-7 | Oral | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 0,5 mg/kg | Non-applicable |
|  | Inhalation | Non-applicable | Non-applicable | 0,35 mg/m³ | Non-applicable |
| Ethylbenzene CAS: 100-41-4 EC: 202-849-4 | Oral | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 180 mg/kg | Non-applicable |
|  | Inhalation | Non-applicable | 293 mg/m³ | 77 mg/m³ | Non-applicable |
| Toluene CAS: 108-88-3 EC: 203-625-9 | Oral | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 384 mg/kg | Non-applicable |
|  | Inhalation | 384 mg/m³ | 384 mg/m³ | 192 mg/m³ | 192 mg/m³ |

DNEL (General population):

|  |  | Short exposure |  | Long exposure |  |
| --- | --- | --- | --- | --- | --- |
| Identification |  | Systemic | Local | Systemic | Local |
| N-butyl acetate CAS: 123-86-4 EC: 204-658-1 | Oral | 2 mg/kg | Non-applicable | 2 mg/kg | Non-applicable |
|  | Dermal | 6 mg/kg | Non-applicable | 6 mg/kg | Non-applicable |
|  | Inhalation | 300 mg/m³ | 300 mg/m³ | 35,7 mg/m³ | 35,7 mg/m³ |
| heptan-2-one CAS: 110-43-0 EC: 203-767-1 | Oral | Non-applicable | Non-applicable | 23,32 mg/kg | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 23,32 mg/kg | Non-applicable |
|  | Inhalation | Non-applicable | Non-applicable | 84,31 mg/m³ | Non-applicable |
| 4-methylpentan-2-one CAS: 108-10-1 EC: 203-550-1 | Oral | Non-applicable | Non-applicable | 4,2 mg/kg | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 4,2 mg/kg | Non-applicable |
|  | Inhalation | 155,2 mg/m³ | 155,2 mg/m³ | 14,7 mg/m³ | 14,7 mg/m³ |
| Xylene CAS: 1330-20-7 EC: 215-535-7 | Oral | Non-applicable | Non-applicable | 12,5 mg/kg | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 125 mg/kg | Non-applicable |
|  | Inhalation | 260 mg/m³ | 260 mg/m³ | 65,3 mg/m³ | 65,3 mg/m³ |
| Hydrocarbons, C9, aromatics CAS: 128601-23-0 EC: 918-668-5 | Oral | Non-applicable | Non-applicable | 11 mg/kg | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 11 mg/kg | Non-applicable |
|  | Inhalation | Non-applicable | Non-applicable | 32 mg/m³ | Non-applicable |

# SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION (continued)

|  |  | Short exposure |  | Long exposure |  |
| --- | --- | --- | --- | --- | --- |
| Identification |  | Systemic | Local | Systemic | Local |
| Dibutyltin Dilaurate CAS: 77-58-7 EC: 201-039-8 | Oral | 0,02 mg/kg | Non-applicable | 0,003 mg/kg | Non-applicable |
|  | Dermal | 0,5 mg/kg | Non-applicable | 0,16 mg/kg | Non-applicable |
|  | Inhalation | 0,04 mg/m³ | Non-applicable | 0,005 mg/m³ | Non-applicable |
| Reaction mass of Bis(1,2,2,6,6-pentamethyl-4-piperidyl) sebacate and Methyl 1,2,2,6,6-pentamethyl-4-piperidyl sebacate CAS: 1065336-91-5 EC: 915-687-0 | Oral | Non-applicable | Non-applicable | 0,05 mg/kg | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 0,25 mg/kg | Non-applicable |
|  | Inhalation | Non-applicable | Non-applicable | 0,17 mg/m³ | Non-applicable |
| Hydroxyphenyl benzotriazol derivative CAS: Non-applicable EC: 400-830-7 | Oral | Non-applicable | Non-applicable | 0,025 mg/kg | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 0,25 mg/kg | Non-applicable |
|  | Inhalation | Non-applicable | Non-applicable | 0,085 mg/m³ | Non-applicable |
| Ethylbenzene CAS: 100-41-4 EC: 202-849-4 | Oral | Non-applicable | Non-applicable | 1,6 mg/kg | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | Non-applicable | Non-applicable |
|  | Inhalation | Non-applicable | Non-applicable | 15 mg/m³ | Non-applicable |
| Toluene CAS: 108-88-3 EC: 203-625-9 | Oral | Non-applicable | Non-applicable | 8,13 mg/kg | Non-applicable |
|  | Dermal | Non-applicable | Non-applicable | 226 mg/kg | Non-applicable |
|  | Inhalation | 226 mg/m³ | 226 mg/m³ | 56,5 mg/m³ | 56,5 mg/m³ |

PNEC:

| Identification |  |  |  |  |
| --- | --- | --- | --- | --- |
| N-butyl acetate CAS: 123-86-4 EC: 204-658-1 | STP | 35,6 mg/L | Fresh water | 0,18 mg/L |
|  | Soil | 0,09 mg/kg | Marine water | 0,018 mg/L |
|  | Intermittent | 0,36 mg/L | Sediment (Fresh water) | 0,981 mg/kg |
|  | Oral | Non-applicable | Sediment (Marine water) | 0,098 mg/kg |
| heptan-2-one CAS: 110-43-0 EC: 203-767-1 | STP | 12,5 mg/L | Fresh water | 0,098 mg/L |
|  | Soil | 0,321 mg/kg | Marine water | 0,01 mg/L |
|  | Intermittent | 0,982 mg/L | Sediment (Fresh water) | 1,89 mg/kg |
|  | Oral | Non-applicable | Sediment (Marine water) | 0,189 mg/kg |
| 4-methylpentan-2-one CAS: 108-10-1 EC: 203-550-1 | STP | 27,5 mg/L | Fresh water | 0,6 mg/L |
|  | Soil | 1,3 mg/kg | Marine water | 0,06 mg/L |
|  | Intermittent | 1,5 mg/L | Sediment (Fresh water) | 8,27 mg/kg |
|  | Oral | Non-applicable | Sediment (Marine water) | 0,83 mg/kg |
| Xylene CAS: 1330-20-7 EC: 215-535-7 | STP | 6,58 mg/L | Fresh water | 0,327 mg/L |
|  | Soil | 2,31 mg/kg | Marine water | 0,327 mg/L |
|  | Intermittent | 0,327 mg/L | Sediment (Fresh water) | 12,46 mg/kg |
|  | Oral | Non-applicable | Sediment (Marine water) | 12,46 mg/kg |
| Dibutyltin Dilaurate CAS: 77-58-7 EC: 201-039-8 | STP | 100 mg/L | Fresh water | 0 mg/L |
|  | Soil | 0,041 mg/kg | Marine water | 0 mg/L |
|  | Intermittent | 0,005 mg/L | Sediment (Fresh water) | 0,05 mg/kg |
|  | Oral | 0,0002 g/kg | Sediment (Marine water) | 0,005 mg/kg |
| Reaction mass of Bis(1,2,2,6,6-pentamethyl-4-piperidyl) sebacate and Methyl 1,2,2,6,6-pentamethyl-4-piperidyl sebacate CAS: 1065336-91-5 EC: 915-687-0 | STP | 1 mg/L | Fresh water | 0,002 mg/L |
|  | Soil | 0,21 mg/kg | Marine water | 0 mg/L |
|  | Intermittent | 0,009 mg/L | Sediment (Fresh water) | 1,05 mg/kg |
|  | Oral | Non-applicable | Sediment (Marine water) | 0,11 mg/kg |
| Hydroxyphenyl benzotriazol derivative CAS: Non-applicable EC: 400-830-7 | STP | 10 mg/L | Fresh water | 0,002 mg/L |
|  | Soil | 2 mg/kg | Marine water | 0 mg/L |
|  | Intermittent | 0,028 mg/L | Sediment (Fresh water) | 3,37 mg/kg |
|  | Oral | Non-applicable | Sediment (Marine water) | 0,337 mg/kg |
| Ethylbenzene CAS: 100-41-4 EC: 202-849-4 | STP | 9,6 mg/L | Fresh water | 0,1 mg/L |
|  | Soil | 2,68 mg/kg | Marine water | 0,01 mg/L |
|  | Intermittent | 0,1 mg/L | Sediment (Fresh water) | 13,7 mg/kg |
|  | Oral | 0,02 g/kg | Sediment (Marine water) | 1,37 mg/kg |

# SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION (continued)

| Identification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Toluene CAS: 108-88-3 EC: 203-625-9 | STP | 13,61 mg/L | Fresh water | 0,68 mg/L |
|  | Soil | 2,89 mg/kg | Marine water | 0,68 mg/L |
|  | Intermittent | 0,68 mg/L | Sediment (Fresh water) | 16,39 mg/kg |
|  | Oral | Non-applicable | Sediment (Marine water) | 16,39 mg/kg |

## Exposure controls:

A.- Individual protection measures, such as personal protective equipment

In accordance with the order of importance to control professional exposure (Directive 98/24/EC) it is recommended to use localized extraction in the work area as a collective protection measure to avoid exceeding the occupational exposure limits. In case of using personal protective equipment it should have CE marking in accordance with Directive 2016/425/EC. For more information on Personal Protective Equipment (storage, use, cleaning, maintenance, class of protection,…) consult the information leaflet provided by the manufacturer. For additional information see subsection 7.1.

All information contained herein is a recommendation which needs some specification from the labour risk prevention services as it is not known whether the company has additional measures at its disposal.

B.- Respiratory protection

| Pictogram | PPE | Labelling | CEN Standard | Remarks |
| --- | --- | --- | --- | --- |
| Mandatory respiratory tract protection | Filter mask for gases and vapours |  | EN 405:2002+A1:2010 | Replace when there is a taste or smell of the contaminant inside the face mask. If the contaminant comes with warnings it is recommended to use isolation equipment. |

C.- Specific protection for the hands

| Pictogram | PPE | Labelling | CEN Standard | Remarks |
| --- | --- | --- | --- | --- |
| Mandatory hand protection | Chemical protective gloves (Material: PVC, Breakthrough time: > 60 min) |  | EN ISO 21420:2020 | Replace the gloves at any sign of deterioration. |

As the product is a mixture of several substances, the resistance of the glove material can not be calculated in advance with total reliability and has therefore to be checked prior to the application.

D.- Eye and face protection

| Pictogram | PPE | Labelling | CEN Standard | Remarks |
| --- | --- | --- | --- | --- |
| Mandatory face protection | Face shield |  | EN 166:2002 EN 167:2002 EN 168:2002 EN ISO 4007:2018 | Clean daily and disinfect periodically according to the manufacturer´s instructions. Use if there is a risk of splashing. |

E.- Body protection

| Pictogram | PPE | Labelling | CEN Standard | Remarks |
| --- | --- | --- | --- | --- |
| Mandatory complete body protection | Disposable clothing for protection against chemical risks, with antistatic and fireproof properties |  | EN 1149-1,2,3 EN 13034:2005+A1:2009 EN ISO 13982- 1:2004/A1:2010 EN ISO 6529:2013 EN ISO 6530:2005 EN ISO 13688:2013 EN 464:1994 | For professional use only. Clean periodically according to the manufacturer´s instructions. |
| Mandatory foot protection | Safety footwear for protection against chemical risk, with antistatic and heat resistant properties |  | EN ISO 13287:2020 EN ISO 20345:2011 EN 13832-1:2019 | Replace boots at any sign of deterioration. |

F.- Additional emergency measures

Environmental exposure controls:

In accordance with the community legislation for the protection of the environment it is recommended to avoid environmental spillage of both the product and its container. For additional information see subsection 7.1.D

Volatile organic compounds:

With regard to Directive 2010/75/EU, this product has the following characteristics:

V.O.C. (Supply):	42,8 % weight

V.O.C. density at 20 ºC:	355 kg/m³ (355 g/L)

Average carbon number:	6,5

Average molecular weight:	112,65 g/mol

SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION (continued)

*Not relevant due to the nature of the product, not providing information property of its hazards.

130 ºC

Non-applicable * 5411,88 Pa (5,41 kPa)

Non-applicable *

983,5 kg/m³ Non-applicable * Non-applicable * Non-applicable * Non-applicable * Non-applicable * Non-applicable * Non-applicable * Non-applicable * Non-applicable * Non-applicable * Non-applicable * Non-applicable *

26 ºC

Liquid Fluid Colourless Chlorine

Non-applicable *

Physical state at 20 ºC: Appearance:

Colour:

Odour:

Odour threshold:

Volatility:

Boiling point at atmospheric pressure: Vapour pressure at 20 ºC:

Vapour pressure at 50 ºC: Evaporation rate at 20 ºC: Product description:

Density at 20 ºC:

Relative density at 20 ºC:

Dynamic viscosity at 20 ºC: Kinematic viscosity at 20 ºC: Kinematic viscosity at 40 ºC: Concentration:

pH:

Vapour density at 20 ºC:

Partition coefficient n-octanol/water 20 ºC: Solubility in water at 20 ºC:

Solubility properties: Decomposition temperature: Melting point/freezing point: Flammability:

Flash Point:

9.1	Information on basic physical and chemical properties:

For complete information see the product datasheet.

Appearance:

SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES

| Emergency measure | Standards | Emergency measure | Standards |
| --- | --- | --- | --- |
|  | ANSI Z358-1 ISO 3864-1:2011, ISO 3864-4:2011 |  | DIN 12 899 ISO 3864-1:2011, ISO 3864-4:2011 |
| Emergency shower |  | Eyewash stations |  |

*Not relevant due to the nature of the product, not providing information property of its hazards.

Non-applicable *

Non-applicable *

Non-applicable * Non-applicable * Non-applicable * Non-applicable *

Non-applicable *

Explosive properties:

Oxidising properties:

Corrosive to metals:

Heat of combustion:

Aerosols-total percentage (by mass) of flammable components:

Other safety characteristics:

Surface tension at 20 ºC: Refraction index:

Information with regard to physical hazard classes:

9.2

Non-applicable * 316 ºC

Not available Not available

Non-applicable

Flammability (solid, gas): Autoignition temperature: Lower flammability limit: Upper flammability limit: Particle characteristics: Median equivalent diameter:

Other information:

SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES (continued)

  - Reactivity:

No hazardous reactions are expected because the product is stable under recommended storage conditions. See section 7.

  - Chemical stability:

Chemically stable under the indicated conditions of storage, handling and use.

  - Possibility of hazardous reactions:

Under the specified conditions, hazardous reactions that lead to excessive temperatures or pressure are not expected.

  - Conditions to avoid:

Applicable for handling and storage at room temperature:

10.5 Incompatible materials:

10.6 Hazardous decomposition products:

Contains substances which require external energy for spontaneous decomposition. Form explosive peroxides when distilled, evaporated or otherwise concentrated.

SECTION 10: STABILITY AND REACTIVITY

11.1 Information on hazard classes as defined in Regulation (EC) No 1272/2008:

The experimental information related to the toxicological properties of the product itself is not available

Dangerous health implications:

In case of exposure that is repetitive, prolonged or at concentrations higher than the recommended occupational exposure limits, adverse effects on health may result, depending on the means of exposure:

A- Ingestion (acute effect):

SECTION 11: TOXICOLOGICAL INFORMATION

| Shock and friction | Contact with air | Increase in temperature | Sunlight | Humidity |
| --- | --- | --- | --- | --- |
| Not applicable | Not applicable | Risk of combustion | Avoid direct impact | Not applicable |

| Acids | Water | Oxidising materials | Combustible materials | Others |
| --- | --- | --- | --- | --- |
| Avoid strong acids | Not applicable | Avoid direct impact | Not applicable | Avoid alkalis or strong bases |

# SECTION 11: TOXICOLOGICAL INFORMATION (continued)

    - Acute toxicity: Based on available data, the classification criteria are not met, however, it contains substances classified as dangerous for consumption. For more information see section 3.

    - Corrosivity/Irritability: Based on available data, the classification criteria are not met. However, it does contain substances classified as hazardous for this effect. For more information see section 3.

B- Inhalation (acute effect):

    - Acute toxicity : Based on available data, the classification criteria are not met. However, it contains substances classified as hazardous for inhalation. For more information see section 3.

    - Corrosivity/Irritability: Based on available data, the classification criteria are not met. However, it contains substances classified as hazardous for inhalation. For more information see section 3.

C- Contact with the skin and the eyes (acute effect):

    - Contact with the skin: Based on available data, the classification criteria are not met. However, it contains substances classified as hazardous for skin contact. For more information see section 3.

    - Contact with the eyes: Produces eye damage after contact.

D- CMR effects (carcinogenicity, mutagenicity and toxicity to reproduction):

    - Carcinogenicity: Exposure to this product can cause cancer. For more specific information on the possible health effects see section 2.

IARC: Hydrocarbons, C9, aromatics (3); Xylene (3); 4-methylpentan-2-one (2B); Ethylbenzene (2B); Toluene (3)

    - Mutagenicity: Exposure to this product can cause genetic modifications. For more specific information on the possible health effects see section 2.

    - Reproductive toxicity: May damage fertility or the unborn child E- Sensitizing effects:

    - Respiratory: Based on available data, the classification criteria are not met, as it does not contain substances classified as hazardous with sensitising effects. For more information see section 3.

    - Skin: Prolonged contact with the skin can result in episodes of allergic contact dermatitis.

F- Specific target organ toxicity (STOT) - single exposure:

Exposure in high concentration can interfere with the central nervous system causing headache, dizziness, vertigo, nausea, vomiting, confusion, and in serious cases, loss of consciousness.

G- Specific target organ toxicity (STOT)-repeated exposure:

    - Specific target organ toxicity (STOT)-repeated exposure: Exposure in high concentration can interfere with the central nervous system causing headache, dizziness, vertigo, nausea, vomiting, confusion, and in serious cases, loss of consciousness.

    - Skin: Repeated exposure may cause skin dryness or cracking H- Aspiration hazard:

Based on available data, the classification criteria are not met. However, it does contain substances classified as hazardous for this effect. For more information see section 3.

## Other information:

Non-applicable

Specific toxicology information on the substances:

| Identification | Acute toxicity |  | Genus |
| --- | --- | --- | --- |
| N-butyl acetate CAS: 123-86-4 EC: 204-658-1 | LD50 oral | 12789 mg/kg | Rat |
|  | LD50 dermal | 14112 mg/kg | Rabbit |
|  | LC50 inhalation | 23,4 mg/L (4 h) | Rat |
| Hydrocarbons, C9, aromatics CAS: 128601-23-0 EC: 918-668-5 | LD50 oral | >2000 mg/kg |  |
|  | LD50 dermal | >2000 mg/kg |  |
|  | LC50 inhalation | >20 mg/L |  |
| Xylene CAS: 1330-20-7 EC: 215-535-7 | LD50 oral | 2100 mg/kg | Rat |
|  | LD50 dermal | 1100 mg/kg | Rat |
|  | LC50 inhalation | 11 mg/L (ATEi) |  |
| heptan-2-one CAS: 110-43-0 EC: 203-767-1 | LD50 oral | 1600 mg/kg | Rat |
|  | LD50 dermal | >2000 mg/kg |  |
|  | LC50 inhalation | 11 mg/L (4 h) | Rat |

| Identification | Acute toxicity |  | Genus |
| --- | --- | --- | --- |
| 4-methylpentan-2-one CAS: 108-10-1 EC: 203-550-1 | LD50 oral | >2000 mg/kg |  |
|  | LD50 dermal | >2000 mg/kg |  |
|  | LC50 inhalation | 11 mg/L (4 h) (ATEi) | Rat |
| Dibutyltin Dilaurate CAS: 77-58-7 EC: 201-039-8 | LD50 oral | 2071 mg/kg | Rat |
|  | LD50 dermal | >2000 mg/kg |  |
|  | LC50 inhalation | >20 mg/L |  |
| Reaction mass of Bis(1,2,2,6,6-pentamethyl-4-piperidyl) sebacate and Methyl 1,2,2,6,6- pentamethyl-4-piperidyl sebacate CAS: 1065336-91-5 EC: 915-687-0 | LD50 oral | 3230 mg/kg | Rat |
|  | LD50 dermal | >2000 mg/kg |  |
|  | LC50 inhalation | >20 mg/L |  |
| Hydroxyphenyl benzotriazol derivative CAS: Non-applicable EC: 400-830-7 | LD50 oral | >2000 mg/kg |  |
|  | LD50 dermal | >2000 mg/kg |  |
|  | LC50 inhalation | >20 mg/L |  |
| Ethylbenzene CAS: 100-41-4 EC: 202-849-4 | LD50 oral | 3500 mg/kg | Rat |
|  | LD50 dermal | 15354 mg/kg | Rabbit |
|  | LC50 inhalation | 17,2 mg/L (4 h) | Rat |
| Toluene CAS: 108-88-3 EC: 203-625-9 | LD50 oral | 5580 mg/kg | Rat |
|  | LD50 dermal | 12124 mg/kg | Rat |
|  | LC50 inhalation | 28,1 mg/L (4 h) | Rat |

| ATE mix |  | Ingredient(s) of unknown toxicity |
| --- | --- | --- |
| Oral | 17877,09 mg/kg (Calculation method) | 0 % |
| Dermal | 34013,66 mg/kg (Calculation method) | 0 % |
| Inhalation | 57,96 mg/L (4 h) (Calculation method) | 0 % |

Acute Toxicity Estimate (ATE mix):

11.2 Information on other hazards:

Endocrine disrupting properties

Endocrine-disrupting properties: The product does not meet the criteria.

Other information

Non-applicable

SECTION 11: TOXICOLOGICAL INFORMATION (continued)

The experimental information related to the eco-toxicological properties of the product itself is not available

12.1 Toxicity:

Acute toxicity:

SECTION 12: ECOLOGICAL INFORMATION

| Identification | Concentration |  | Species | Genus |
| --- | --- | --- | --- | --- |
| N-butyl acetate | LC50 | Non-applicable |  |  |
| CAS: 123-86-4 | EC50 | Non-applicable |  |  |
| EC: 204-658-1 | EC50 | 675 mg/L (72 h) | Scenedesmus subspicatus | Algae |
| heptan-2-one | LC50 | 131 mg/L (96 h) | Pimephales promelas | Fish |
| CAS: 110-43-0 | EC50 | Non-applicable |  |  |
| EC: 203-767-1 | EC50 | Non-applicable |  |  |
| 4-methylpentan-2-one | LC50 | 900 mg/L (48 h) | Leuciscus idus | Fish |
| CAS: 108-10-1 | EC50 | 862 mg/L (24 h) | Daphnia magna | Crustacean |
| EC: 203-550-1 | EC50 | 980 mg/L (48 h) | Scenedesmus subspicatus | Algae |
| Xylene | LC50 | >10 - 100 mg/L (96 h) |  | Fish |
| CAS: 1330-20-7 | EC50 | >10 - 100 mg/L (48 h) |  | Crustacean |
| EC: 215-535-7 | EC50 | >10 - 100 mg/L (72 h) |  | Algae |

# CTION 12: ECOLOGICAL INFORMATION (continued)

| Identification | Concentration |  | Species | Genus |
| --- | --- | --- | --- | --- |
| Hydrocarbons, C9, aromatics | LC50 | >1 - 10 mg/L (96 h) |  | Fish |
| CAS: 128601-23-0 | EC50 | >1 - 10 mg/L (48 h) |  | Crustacean |
| EC: 918-668-5 | EC50 | >1 - 10 mg/L (72 h) |  | Algae |
| Dibutyltin Dilaurate | LC50 | >0.1 - 1 mg/L (96 h) |  | Fish |
| CAS: 77-58-7 | EC50 | >0.1 - 1 mg/L (48 h) |  | Crustacean |
| EC: 201-039-8 | EC50 | >0.1 - 1 mg/L (72 h) |  | Algae |
| Reaction mass of Bis(1,2,2,6,6-pentamethyl-4-piperidyl) sebacate | LC50 | 0,9 mg/L (96 h) | Danio rerio | Fish |
| and Methyl 1,2,2,6,6-pentamethyl-4-piperidyl sebacate |  |  |  |  |
| CAS: 1065336-91-5 | EC50 | Non-applicable |  |  |
| EC: 915-687-0 | EC50 | 1,7 mg/L (72 h) | Desmodesmus subspicatus | Algae |
| Hydroxyphenyl benzotriazol derivative CAS: Non-applicable EC: 400-830-7 | LC50 | >1 - 10 mg/L (96 h) |  | Fish |
|  | EC50 | >1 - 10 mg/L (48 h) |  | Crustacean |
|  | EC50 | >1 - 10 mg/L (72 h) |  | Algae |
| Ethylbenzene | LC50 | 42,3 mg/L (96 h) | Pimephales promelas | Fish |
| CAS: 100-41-4 | EC50 | 75 mg/L (48 h) | Daphnia magna | Crustacean |
| EC: 202-849-4 | EC50 | 63 mg/L (3 h) | Chlorella vulgaris | Algae |
| Toluene | LC50 | 13 mg/L (96 h) | Carassius auratus | Fish |
| CAS: 108-88-3 | EC50 | 11,5 mg/L (48 h) | Daphnia magna | Crustacean |
| EC: 203-625-9 | EC50 | Non-applicable |  |  |

Chronic toxicity:

| Identification | Concentration |  | Species | Genus |
| --- | --- | --- | --- | --- |
| N-butyl acetate CAS: 123-86-4 EC: 204-658-1 | NOEC | Non-applicable |  |  |
|  | NOEC | 23,2 mg/L | Daphnia magna | Crustacean |
| 4-methylpentan-2-one CAS: 108-10-1 EC: 203-550-1 | NOEC | Non-applicable |  |  |
|  | NOEC | 78 mg/L | Daphnia magna | Crustacean |
| Xylene CAS: 1330-20-7 EC: 215-535-7 | NOEC | 1,3 mg/L | Oncorhynchus mykiss | Fish |
|  | NOEC | 1,17 mg/L | Ceriodaphnia dubia | Crustacean |
| Reaction mass of Bis(1,2,2,6,6-pentamethyl-4-piperidyl) sebacate and Methyl 1,2,2,6,6-pentamethyl-4-piperidyl sebacate CAS: 1065336-91-5 EC: 915-687-0 | NOEC | Non-applicable |  |  |
|  | NOEC | 1 mg/L | Daphnia magna | Crustacean |
| Ethylbenzene CAS: 100-41-4 EC: 202-849-4 | NOEC | Non-applicable |  |  |
|  | NOEC | 0,96 mg/L | Ceriodaphnia dubia | Crustacean |

  - Persistence and degradability: Substance-specific information:

| Identification | Degradability |  | Biodegradability |  |
| --- | --- | --- | --- | --- |
| N-butyl acetate CAS: 123-86-4 EC: 204-658-1 | BOD5 | Non-applicable | Concentration | Non-applicable |
|  | COD | Non-applicable | Period | 5 days |
|  | BOD5/COD | Non-applicable | % Biodegradable | 84 % |
| 4-methylpentan-2-one CAS: 108-10-1 EC: 203-550-1 | BOD5 | 2,06 g O2/g | Concentration | 100 mg/L |
|  | COD | 2,16 g O2/g | Period | 14 days |
|  | BOD5/COD | 0,95 | % Biodegradable | 84 % |
| Xylene CAS: 1330-20-7 EC: 215-535-7 | BOD5 | Non-applicable | Concentration | Non-applicable |
|  | COD | Non-applicable | Period | 28 days |
|  | BOD5/COD | Non-applicable | % Biodegradable | 88 % |
| Dibutyltin Dilaurate CAS: 77-58-7 EC: 201-039-8 | BOD5 | 0 g O2/g | Concentration | 100 mg/L |
|  | COD | Non-applicable | Period | 28 days |
|  | BOD5/COD | Non-applicable | % Biodegradable | 50 % |
| Reaction mass of Bis(1,2,2,6,6-pentamethyl-4-piperidyl) sebacate and Methyl 1,2,2,6,6-pentamethyl-4-piperidyl sebacate CAS: 1065336-91-5 EC: 915-687-0 | BOD5 | Non-applicable | Concentration | 20 mg/L |
|  | COD | Non-applicable | Period | 28 days |
|  | BOD5/COD | Non-applicable | % Biodegradable | 38 % |

# SECTION 12: ECOLOGICAL INFORMATION (continued)

| Identification | Degradability |  | Biodegradability |  |
| --- | --- | --- | --- | --- |
| Ethylbenzene CAS: 100-41-4 EC: 202-849-4 | BOD5 | Non-applicable | Concentration | 100 mg/L |
|  | COD | Non-applicable | Period | 14 days |
|  | BOD5/COD | Non-applicable | % Biodegradable | 90 % |
| Toluene CAS: 108-88-3 EC: 203-625-9 | BOD5 | 2,5 g O2/g | Concentration | 100 mg/L |
|  | COD | Non-applicable | Period | 14 days |
|  | BOD5/COD | Non-applicable | % Biodegradable | 100 % |

  - Bioaccumulative potential: Substance-specific information:

| Identification | Bioaccumulation potential |  |
| --- | --- | --- |
| N-butyl acetate | BCF | 4 |
| CAS: 123-86-4 | Pow Log | 1.78 |
| EC: 204-658-1 | Potential | Low |
| heptan-2-one | BCF | 7 |
| CAS: 110-43-0 | Pow Log | 1.98 |
| EC: 203-767-1 | Potential | Low |
| 4-methylpentan-2-one | BCF | 2 |
| CAS: 108-10-1 | Pow Log | 1.31 |
| EC: 203-550-1 | Potential | Low |
| Xylene | BCF | 9 |
| CAS: 1330-20-7 | Pow Log | 2.77 |
| EC: 215-535-7 | Potential | Low |
| Dibutyltin Dilaurate | BCF | 31 |
| CAS: 77-58-7 | Pow Log | 3.12 |
| EC: 201-039-8 | Potential | Moderate |
| Ethylbenzene | BCF | 1 |
| CAS: 100-41-4 | Pow Log | 3.15 |
| EC: 202-849-4 | Potential | Low |
| Toluene | BCF | 90 |
| CAS: 108-88-3 | Pow Log | 2.73 |
| EC: 203-625-9 | Potential | Moderate |

  - Mobility in soil:

| Identification | Absorption/desorption |  | Volatility |  |
| --- | --- | --- | --- | --- |
| N-butyl acetate CAS: 123-86-4 EC: 204-658-1 | Koc | Non-applicable | Henry | Non-applicable |
|  | Conclusion | Non-applicable | Dry soil | Non-applicable |
|  | Surface tension | 2,478E-2 N/m (25 ºC) | Moist soil | Non-applicable |
| heptan-2-one CAS: 110-43-0 EC: 203-767-1 | Koc | 280 | Henry | 17,12 Pa·m³/mol |
|  | Conclusion | Moderate | Dry soil | Yes |
|  | Surface tension | 2,612E-2 N/m (25 ºC) | Moist soil | Yes |
| 4-methylpentan-2-one CAS: 108-10-1 EC: 203-550-1 | Koc | Non-applicable | Henry | Non-applicable |
|  | Conclusion | Non-applicable | Dry soil | Non-applicable |
|  | Surface tension | 2,35E-2 N/m (25 ºC) | Moist soil | Non-applicable |
| Xylene CAS: 1330-20-7 EC: 215-535-7 | Koc | 202 | Henry | 524,86 Pa·m³/mol |
|  | Conclusion | Moderate | Dry soil | Yes |
|  | Surface tension | Non-applicable | Moist soil | Yes |
| Reaction mass of Bis(1,2,2,6,6-pentamethyl-4-piperidyl) sebacate and Methyl 1,2,2,6,6-pentamethyl-4-piperidyl sebacate CAS: 1065336-91-5 EC: 915-687-0 | Koc | 204400 | Henry | 0E+0 Pa·m³/mol |
|  | Conclusion | Immobile | Dry soil | No |
|  | Surface tension | Non-applicable | Moist soil | No |

163, 367, 650

D/E

see section 9

5 L

Non-applicable

UN1263 PAINT 3

3

III

No

UN number or ID number: UN proper shipping name: Transport hazard class(es): Labels:

Packing group: Environmental hazards: Special precautions for user Special regulations:

Tunnel restriction code: Physico-Chemical properties: Limited quantities:

Maritime transport in bulk according to IMO instruments:

14.1

14.2

14.3

14.4

14.5

14.6

14.7

Transport of dangerous goods by land:

With regard to ADR 2023 and RID 2023:

SECTION 14: TRANSPORT INFORMATION

| Identification | Absorption/desorption |  | Volatility |  |
| --- | --- | --- | --- | --- |
| Ethylbenzene CAS: 100-41-4 EC: 202-849-4 | Koc | 520 | Henry | 798,44 Pa·m³/mol |
|  | Conclusion | Moderate | Dry soil | Yes |
|  | Surface tension | 2,859E-2 N/m (25 ºC) | Moist soil | Yes |
| Toluene CAS: 108-88-3 EC: 203-625-9 | Koc | 178 | Henry | 672,8 Pa·m³/mol |
|  | Conclusion | Moderate | Dry soil | Yes |
|  | Surface tension | 2,793E-2 N/m (25 ºC) | Moist soil | Yes |

  - Results of PBT and vPvB assessment:

Product does not meet PBT/vPvB criteria

  - Endocrine disrupting properties:

Endocrine-disrupting properties: The product does not meet the criteria.

  - Other adverse effects:

Not described

SECTION 12: ECOLOGICAL INFORMATION ** (continued)

13.1 Waste treatment methods:

Type of waste (Regulation (EU) No 1357/2014):

HP14 Ecotoxic, HP3 Flammable, HP5 Specific Target Organ Toxicity (STOT)/Aspiration Toxicity, HP7 Carcinogenic, HP10 Toxic for reproduction, HP11 Mutagenic

Waste management (disposal and evaluation):

Consult the authorized waste service manager on the assessment and disposal operations in accordance with Annex 1 and Annex 2 (Directive 2008/98/EC). As under 15 01 (2014/955/EC) of the code and in case the container has been in direct contact with the product, it will be processed the same way as the actual product. Otherwise, it will be processed as non-dangerous residue. Waste should not be disposed of to drains. See paragraph 6.2.

Regulations related to waste management:

In accordance with Annex II of Regulation (EC) No 1907/2006 (REACH) the community or state provisions related to waste management are stated

Community legislation: Directive 2008/98/EC, 2014/955/EU, Regulation (EU) No 1357/2014

SECTION 13: DISPOSAL CONSIDERATIONS

| Code | Description | Waste class (Regulation (EU) No 1357/2014) |
| --- | --- | --- |
| 08 01 11* | waste paint and varnish containing organic solvents or other hazardous substances | Dangerous |
| 15 01 10* | packaging containing residues of or contaminated by hazardous substances |  |

see section 9

Non-applicable

UN1263 PAINT 3

3

III

No

UN number or ID number: UN proper shipping name: Transport hazard class(es): Labels:

Packing group: Environmental hazards: Special precautions for user

Physico-Chemical properties:

Maritime transport in bulk according to IMO instruments:

14.1

14.2

14.3

14.4

14.5

14.6

14.7

Transport of dangerous goods by air:

With regard to IATA/ICAO 2023:

UN1263 PAINT 3

3

III

No

223, 955, 163, 367

F-E, S-E

see section 9

5 L

Non-applicable

Non-applicable

UN number or ID number: UN proper shipping name: Transport hazard class(es): Labels:

Packing group: Marine pollutant:

Special precautions for user

Special regulations:

EmS Codes:

Physico-Chemical properties: Limited quantities: Segregation group:

Maritime transport in bulk according to IMO instruments:

14.1

14.2

14.3

14.4

14.5

14.6

14.7

Transport of dangerous goods by sea:

With regard to IMDG 40-20:

SECTION 14: TRANSPORT INFORMATION (continued)

15.1 Safety, health and environmental regulations/legislation specific for the substance or mixture: Candidate substances for authorisation under the Regulation (EC) No 1907/2006 (REACH): Non-applicable Substances included in Annex XIV of REACH ("Authorisation List") and sunset date: Non-applicable Regulation (EC) No 1005/2009, about substances that deplete the ozone layer: Non-applicable

Article 95, REGULATION (EU) No 528/2012: Non-applicable

REGULATION (EU) No 649/2012, in relation to the import and export of hazardous chemical products: Contains Dibutyltin Dilaurate

Seveso III:

Limitations to commercialisation and the use of certain dangerous substances and mixtures (Annex XVII REACH, etc ….):

SECTION 15: REGULATORY INFORMATION

| Section | Description | Lower-tier requirements | Upper-tier requirements |
| --- | --- | --- | --- |
| P5c | FLAMMABLE LIQUIDS | 5000 | 50000 |

# SECTION 15: REGULATORY INFORMATION (continued)

Product classified hazardous under the CMR. Sale and distribution to the general public is prohibited. Due to its CMR category, it is essential to apply the specific measures for workplace hazard prevention covered in articles 4 and 5 of the 2004/37/EC Directive and later modifications.

Contains more than 0.5347 % of Dibutyltin Dilaurate by weight. Dibutylester (DBM) compounds may not be used after 1st January 2012 in mixtures and articles intended for the general public when concentration in said mixture or article or in any part of the same is greater than the equivalent of 0.1 % of tin by weight. Shall not be placed on the market, or used, as substances or in mixtures where the substance or mixture is acting as biocide in free association paint. Shall not be placed on the market, or used, as substances or in mixtures where the substance or mixture acts as biocide to prevent the fouling by micro-organisms, plants or animals of: (a) all craft irrespective of their length intended for use in marine, coastal, estuarine and inland waterways and lakes (b) cages, floats, nets and any other appliances or equipment used for fish or shellfish farming (c) any totally or partly submerged appliance or equipment. Shall not be placed on the market, or used, as substances or in mixtures where the substance or mixture is intended for use in the treatment of industrial waters.

Shall not be used in:

—ornamental articles intended to produce light or colour effects by means of different phases, for example in ornamental lamps and ashtrays,

—tricks and jokes,

—games for one or more participants, or any article intended to be used as such, even with ornamental aspects.

## Specific provisions in terms of protecting people or the environment:

It is recommended to use the information included in this safety data sheet as a basis for conducting workplace-specific risk assessments in order to establish the necessary risk prevention measures for the handling, use, storage and disposal of this product.

## Other legislation:

The product could be affected by sectorial legislation

## 15.2 Chemical safety assessment:

The supplier has not carried out evaluation of chemical safety.

# SECTION 16: OTHER INFORMATION

## Legislation related to safety data sheets:

The SDS shall be supplied in an official language of the country where the product is placed on the market. This safety data sheet has been designed in accordance with ANNEX II-Guide to the compilation of safety data sheets of Regulation (EC) No 1907/2006 (COMMISSION REGULATION (EU) 2020/878).

## Modifications related to the previous Safety Data Sheet which concerns the ways of managing risks.:

COMPOSITION/INFORMATION ON INGREDIENTS (SECTION 3, SECTION 11, SECTION 12):

· New declared substances Ethylbenzene (100-41-4)

Hydroxyphenyl benzotriazol derivative Dibutyltin Dilaurate (77-58-7)

Reaction mass of Bis(1,2,2,6,6-pentamethyl-4-piperidyl) sebacate and Methyl 1,2,2,6,6-pentamethyl-4-piperidyl sebacate (1065336-91-5)

Toluene (108-88-3)

- Removed substances

Bis(1,2,2,6,6-pentamethyl-4-piperidyl) sebacate (41556-26-7) Methyl 1,2,2,6,6-pentamethyl-4-piperidyl sebacate (82919-37-7)

Substances that contribute to the classification (SECTION 2):

- New declared substances

Reaction mass of Bis(1,2,2,6,6-pentamethyl-4-piperidyl) sebacate and Methyl 1,2,2,6,6-pentamethyl-4-piperidyl sebacate (1065336-91-5)

CLP Regulation (EC) No 1272/2008 (SECTION 2, SECTION 16):

- Hazard statements

- Precautionary statements

- Substances contained in EUH208:

  - New declared substances Hydroxyphenyl benzotriazol derivative Dibutyltin Dilaurate (77-58-7)

  - Removed substances

Bis(1,2,2,6,6-pentamethyl-4-piperidyl) sebacate (41556-26-7) Methyl 1,2,2,6,6-pentamethyl-4-piperidyl sebacate (82919-37-7)

Texts of the legislative phrases mentioned in section 2:

# SECTION 16: OTHER INFORMATION (continued)

H336: May cause drowsiness or dizziness.

H412: Harmful to aquatic life with long lasting effects.

H373: May cause damage to organs through prolonged or repeated exposure (Oral). H351: Suspected of causing cancer.

H317: May cause an allergic skin reaction. H341: Suspected of causing genetic defects. H360: May damage fertility or the unborn child. H371: May cause damage to organs.

H226: Flammable liquid and vapour. H319: Causes serious eye irritation.

## Texts of the legislative phrases mentioned in section 3:

The phrases indicated do not refer to the product itself; they are present merely for informative purposes and refer to the individual components which appear in section 3

## CLP Regulation (EC) No 1272/2008:

Acute Tox. 4: H302+H332 - Harmful if swallowed or if inhaled.

Acute Tox. 4: H312+H332 - Harmful in contact with skin or if inhaled. Acute Tox. 4: H332 - Harmful if inhaled.

Aquatic Acute 1: H400 - Very toxic to aquatic life.

Aquatic Chronic 1: H410 - Very toxic to aquatic life with long lasting effects. Aquatic Chronic 2: H411 - Toxic to aquatic life with long lasting effects.

Aquatic Chronic 3: H412 - Harmful to aquatic life with long lasting effects. Asp. Tox. 1: H304 - May be fatal if swallowed and enters airways.

Carc. 2: H351 - Suspected of causing cancer. Eye Irrit. 2: H319 - Causes serious eye irritation.

Flam. Liq. 2: H225 - Highly flammable liquid and vapour. Flam. Liq. 3: H226 - Flammable liquid and vapour.

Muta. 2: H341 - Suspected of causing genetic defects. Repr. 1B: H360 - May damage fertility or the unborn child. Repr. 2: H361d - Suspected of damaging the unborn child. Repr. 2: H361f - Suspected of damaging fertility.

Skin Irrit. 2: H315 - Causes skin irritation.

Skin Sens. 1: H317 - May cause an allergic skin reaction. Skin Sens. 1A: H317 - May cause an allergic skin reaction.

STOT RE 1: H372 - Causes damage to organs through prolonged or repeated exposure. (Oral). STOT RE 2: H373 - May cause damage to organs through prolonged or repeated exposure (Oral). STOT RE 2: H373 - May cause damage to organs through prolonged or repeated exposure.

STOT SE 1: H370 - Causes damage to organs. STOT SE 3: H335 - May cause respiratory irritation.

STOT SE 3: H336 - May cause drowsiness or dizziness.

## Classification procedure:

STOT SE 3: Calculation method Aquatic Chronic 3: Calculation method STOT RE 2: Calculation method

Carc. 2: Calculation method

Skin Sens. 1A: Calculation method Muta. 2: Calculation method

Repr. 1B: Calculation method STOT SE 2: Calculation method

Flam. Liq. 3: Calculation method (2.6.4.3) Eye Irrit. 2: Calculation method

## Advice related to training:

Training is recommended in order to prevent industrial risks for staff using this product and to facilitate their comprehension and interpretation of this safety data sheet, as well as the label on the product.

## Principal bibliographical sources:

http://echa.europa.eu http://eur-lex.europa.eu

Abbreviations and acronyms:

ADR: European agreement concerning the international carriage of dangerous goods by road IMDG: International maritime dangerous goods code

IATA: International Air Transport Association ICAO: International Civil Aviation Organisation COD: Chemical Oxygen Demand

BOD5: 5day biochemical oxygen demand BCF: Bioconcentration factor

LD50: Lethal Dose 50

LC50: Lethal Concentration 50 EC50: Effective concentration 50

LogPOW: Octanolwater partition coefficient Koc: Partition coefficient of organic carbon UFI: unique formula identifier

IARC: International Agency for Research on Cancer

SECTION 16: OTHER INFORMATION (continued)

The information contained in this safety data sheet is based on sources, technical knowledge and current legislation at European and state level, without being able to guarantee its accuracy. This information cannot be considered a guarantee of the properties of the product, it is simply a description of the security requirements. The occupational methodology and conditions for users of this product are not within our awareness or control, and it is ultimately the responsibility of the user to take the necessary measures to obtain the legal requirements concerning the manipulation, storage, use and disposal of chemical products. The information on this safety data sheet only refers to this product, which should not be used for needs other than those specified.

- END OF SAFETY DATA SHEET -