TECHNICAL DATA SHEET

HARDENER MS CLEARCOAT PLUS 112

Product codes: 80904  0,5L normal

80905  0,5L fast

80906  0,25L normal

| # PRODUCT DESCRIPTION |  |
| --- | --- |
| This is a special hardener, composed for MS Clearcoat PLUS 112. Application in a ratio of 1:2 with clearcoat. |  |
| # TYPES OF HARDENER |  |
| The use of the appropriate type of hardener depends on the room temperature Slow Normal Fast 35°C Rate of hardening 18°C | - normal (whole cars) |
|  | - fast (1 - 2 elements) |
| The type of used hardener affects coating drying time and depends on room temperature and the preference of the painter. Proper selection of the type of hardener ensures appropriate structure and appearance of coating. |  |
| # RULES OF CONDUCT |  |
| Please note that after opening the container with a hardener and mixing it with clearcoat, you should immediately close it to prevent the hardening reaction in contact with air humidity. |  |
| # STORAGE TIME AND CONDITIONS |  |
| Store in tightly closed containers, in a cool, dry place away from sources of fire and heat. Avoid exposure to sunlight. Expiration date: 12 months from date of manufacture - date on the top of container. |  |
| # CLEANING EQUIPMENT |  |
| For cleaning equipment can be used acryl thinner or nitro thinner. It is recommended to use thinner for cleaning paint spray guns. |  |
| # HEALTH AND SAFETY REGULATIONS |  |
| See Safety Data Sheet of the product. |  |
| # SPECIFIC WEIGHT |  |
| About 1.0 g/cm3 |  |
| OTHER INFORMATION The effectiveness of our systems results from laboratory research and many years of experience. The data contained herein meets the current knowledge about our products and their application potential. We ensure high quality, provided the user follows the instructions and the work is performed in accordance with good workmanship. It is necessary to do a test application of the product due to its potentially different reaction with different materials. We may not be held liable for defects if the final result was affected by factors beyond our control. RAGS Ltd., Reg. No.: 40003133752, VAT number: LV40003133752 Legal address: Durbes street 8, Rīga, LV-1007, Actual address: Džūkstes street 1, Rīga, LV-1004 Tel. +371 67 808 780, +371 26 542 777. |  |