Stonder Glue Spray

Technical Information

# Quality & properties
Use STONDER GLUE to glue paper, cardboard, wood, textiles, cork, leather and various types of plastic for both permanent and temporary adhesion.

# Physical & chemical data

Suitable to glue large surfaces

Spray pattern about 10 centimetre wide Fast drying

User-friendly

Base: Synthetic resin Colourname: transparent white Smell: Characteristic

Surfaces: Paper, Cardboard, Wood, Textile, Cork, Leather, Various types of plastic Minimum Working Temperature: 10 °C Maximum Working Temperature: 25 °C Spray rate: 1,2

Vapour pressure: 4000/20 bar Relative density: 0.75 g/ml Storage stability: 48 months Content: 400 ml

# How to use

Before use, carefully read the directions on the packaging and act accordingly.

The surface should be clean, dry and free of grease. The aerosol should have room temperature. Best processing temperature 10 to 25°C. Before use, shake the aerosol.

Distance to the surface to be treated approximately 25 to 30 centimetre. Apply glue in an equal layer.

For a permanent adhesion, treat both parts to be glued, allow to dry 15 to 20 minutes and firmly press together.

For a temporary adhesion, treat one of the parts to be glued, allow to dry for 5 to 10 minutes and firmly press together.

When treating porous surfaces, apply several layers.

After use, clean the valve (turn aerosol upside down and press the nozzle for approximately 5 seconds)

Other information

The effectiveness of our systems results from laboratory research and many years of experience. The data contained herein meets the current knowledge about our products and their application potential. We ensure high quality, provided the user follows the instructions and the work is performed in accordance with good workmanship. It is necessary to do a test application of the product due to its potentially different reaction with different materials. We may not be held liable for defects if the final result was affected by factors beyond our control.

RAGS Ltd.,  Reg. No.: 40003133752, VAT number: LV40003133752

Legal address: Durbes street 8, Rīga, LV-1007,

Actual address: Džūkstes street 1, Rīga, LV-1004

Tel. +371 67 808 780, +371 26 542 777